# Application and characterization of blueberry fruit pigment as photosensitizer for high performance and stable natural dye sensitized solid state solar cells

Asitha Malikaramge1, Prabawathy Nagarajan2, R.M.G. Rjapakse1, G.R.A. Kumara3,

Dhayalan Velauthapillai4, Punniamoorthy Ravirajan5

1 Postgraduate Institute of Science, University of Peradeniya, Peradeniya 20400,  Sri Lanka.

2PSG College of Technology,  Department of Physics & Material Sciences,· Coimbatore, India

3National Institute of Fundamental Studies, Hantana Road,Kandy 20000, Sri Lanka.

4Høgskulen på Vestlandet,  Faculty of Science and Engineering,  Bergen, Hordaland, Norway

5Departmet of Physics, University of Jaffna, Jaffna  40 000, Sri Lanka.

### Abstract

The natural dye pigment of blueberry was extracted from its peals using acetonitrile as solvent. This dye was used in a liquid electrolyte-free, natural dye-sensitized solid solar cell (NDSSSC). Natural dyes are inexpensive, non-toxic and are reliable and redily available sources of dyes. However, the typical problem associated with them is their instability towards iodide/tri-iodide electrolyte. This was addressed by introducing p-CuI as the hole-conductor in place of iodide/tri-iodide electrolyte. The hole conductor was applied on dyed nonporous TiO2 films from a solution containing a crystal growth inhibitor triethylammium thiocyanate (THT) using drop-casting method. I-V characteristics and impedance measurements were carried out to   investigate the photovoltaic performance and further characterized by UV-Visible spectroscopy, FTIR spectroscopy and SEM.  to achieve a highest efficiency(ɳ) of 1.7% with a short circuit current density (Jsc) of 11 mA cm-2, open circuit voltage (Voc) of 0.30 V and fill factor (ff) of 51.8. These are the highest recorded values so far achieved for such solar cells. Stability measurements were carried out for a period of 30 days and promisingly showed a good stability over the liquid type natural dye sensitised solar.

Keywords— Dye-sensitized Solid-State Solar Cell, Bluberry Dye, Copper Iodide Hole Conductor, Natural Dye

Recent Publications

- Prabavathy, N., Balasundaraprabhu, R., Balaji, G., Malikaramage, A. U. & Prasanna, S. Journal of Photochemistry & Photobiology A : Chemistry Investigations on the photo catalytic activity of calcium doped TiO 2 photo electrode for enhanced e ffi ciency of anthocyanins based dye sensitized solar cells. J. Photochem. Photobiol. A Chem. 377, 43–57 (2019).

Biography:

Asitha Udayanga Malikaramage holds a B. Sc. Degree in Chemistry and Physics stream from University of Peradeniya, Sri Lanka, and currently he is reading for his M.Phil. Degree attached to the Postgraduate Institute of Science, University of Peradeniya. His research focuses on photon upconversion as a tool to harvest infrared radiation for direct illumination in the dark and to fabricate dye-sensitized solar cells to generate electricity under illumination as well as in the dark and is carried out under the supervision of the co-authors of this abstract. He has one international publication and a several communications at international conferences to his credit and he is currently engaged with student mobility programme between Sri Lanka and Norway.

Presenting author details:

Full Name : Asitha Udayanga Malikaramage

Email Id : asitha4htc@gmail.com

Contact No: +94719153257

Social Network Details-Facebook : https://www.facebook.com/asitha.udayanga

Any Comments:

Acknowledgement

National science Foundation Sri Lanka Grant no RG/2016/BS/01