Energy storage materials for supercapacitor applications

S.Selladurai and P.Saranya

Department of Physics, Anna University, Chennai-25, India

Energy has become primary focus for modernistic applications such as electronic gadgets, hybrid electric vehicles and instant switches. Efficient energy storage devices are more important for the developing world to overcome energy shortage. Supercapacitors has placed high attraction for electronic applications than batteries. Compared to batteries, supercapacitors has high power density, long cycle life, it experience no memory, charging circuit is simple and improving energy density than conventional capacitors.Supercapacitors electrode materials synthesised using metal oxides and graphene material provides excellent specific capacitance than carbon based materials. Graphene is an outstanding electrode material for supercapacitors owing to its high electrical conductivity, high surface area and fast redox reactions. Metal oxides chemically integrated with graphene yields double the times of specific capacitance compared to metal oxides.Current research is to overcome the lack of energy density in supercapacitors.Based on the literature the electrochemical performance of binary metal oxides is higher than monometallic oxides. When two metal oxides conjoined to form spinel structure or composite, its specific capacitance enhances due to richer redox activity from two different metal ion species. Our group has worked on various metal oxides such as NiO, CeO2, NiO-CeO2 composite, ZnO, Co3O4, NiCo2O4, MnCo2O4, CoMoO4, Mn doped cobalt oxide, ZnMn2O4. Graphene incorporated metal oxides such as Co3O4– GNS, ZnMn2O4/rGO, MnCo2O4 – GNS. These nanomaterials were synthesized by various techniques such as sol-gel, co-precipitation, hydrothermal and combustion method. Various nanostructured particles provide advantages during intercalation process of electrode. Electrochemical performance of these nanomaterials were studied in different aqueous electrolytes using cyclic voltammetry, charge-charge and electrochemical impedance analysis. The resulted specific capacitance values were tabulated in below table. Our aim is to attain high energy density electrode materials for supercapacitors.

Objectives:  (i) Prepare the new metal oxide, graphene based energy storage materials for supercapacitor application  (ii) Study their electrochemical properties for device applications

(iii) Prototype device fabricated and test their capacity

Approach:	(i) Supercapacitors has high power density

(ii) Long cycle life, it experience no memory, charging circuit is simple and improving energy density than conventional capacitors

(iii) Supercapacitors electrode materials synthesised using metal oxides and graphene material provides excellent specific capacitance than carbon based materials.

(iv) Graphene is an outstanding electrode material for supercapacitors owing to its high electrical conductivity, high surface area and fast redox reactions.

(v) Metal oxides chemically integrated with graphene yields double the times of specific capacitance compared to metal oxides.

(vi) Current research is to overcome the lack of energy density in supercapacitors.

(vii) Based on the literature the electrochemical performance of binary metal oxides is higher than monometallic oxides. When two metal oxides conjoined to form spinel structure or composite, its specific capacitance enhances due to richer redox activity from two different metal ion species.

(viii) Based Materials are NiO, CeO2, NiO-CeO2 composite, ZnO, Co3O4, NiCo2O4, MnCo2O4, CoMoO4, Mn doped cobalt oxide, ZnMn2O4. Graphene incorporated metal oxides such as Co3O4– GNS, ZnMn2O4/rGO, MnCo2O4 – GNS. These nanomaterials were synthesized by various techniques such as sol-gel, co-precipitation, hydrothermal and combustion method.

Deliverable: (i) Fabricate the supercapacitor in prototype

(ii) Study their capacitance (iii) Study their life and also test it in the energy discharge.

Impact: Alternate for Lithium ion batteries, Environmentally safe, materials are available easily.