HEAVY METALS SPECIATION OF OTIN RIVER IN OSUN STATE NIGERIA

Adetoro R.O.1*, Adekola F.A.2,  Adebisi S.A.1, Azeez L.A.1 Adejumo A.L.1

1-Department of Chemical Sciences, Osun State University, Osogbo, Nigeria.

2-Department of Industrial Chemistry, University of Ilorin, Ilorin, Nigeria.

rasheed.adetoro@uniosun.edu.ng

+2348034175769

ABSTRACT

As a result of rapid urbanisation of major towns in Osun State since its creation in 1991, the activities around the rivers have increased tremendously with little or no regard for environmentally sound practice. Total concentrations of Mn, Fe, Cu, Zn, Cr, Pb and Ni in sediments were determined using wet digestion and atomic absorption spectrophotometric methods (AAS). Speciation of the elements was carried out using a five sequential extraction method. Results obtained revealed that In Otin sediments, the exchangeable fraction followed the trend: Cd > Cu > Mn > Zn > Fe while Cr and Pb were not detected. Carbonate fraction followed the trend: Cu > Zn > Cd > Mn > Fe while Cr and Pb were not detected. Reducible fraction was: Mn > Pb > Zn > Fe > Cu > Cr while Cd was not detected. Organic fraction was: Mn > Cu > Zn > Cr > Fe while Cd and Pb were not detected. Residual fraction was: Cr >Fe >Pb > Cu > Zn > Mn > Cd. It is therefore recommended that; The high percentage of majority of metals i.e. Fe, Cu, Zn, Cr and Pb (except upper course) in residual fractions (non-bioavailable form) in all the sampling sites for Otin river showed that the sediments could not be used for construction works (bridges and buildings). The non-bioavailability of Fe, Cu, Zn, Cr and Pb (except upper course) posed a much lower risk to the water body in normal conditions because of their low mobility in the sediment environment.

Keywords: speciation, sediments, heavy metals, bioavailability, non- bioavailability