# A multi-analytical approach to assess the impact of air pollution on cultural heritage: degradation of stones, mortars and bricks

Paola Fermo, Valeria Comite

Dipartimento di Chimica, Università degli Studi di Milano, Milan (Italy)

### Abstract (300 word limit)

Air pollution is one of the most important causes of surface decay in urban environment. Among the degradation processes due to airborne pollutants, the formation of black crusts is one of the most dangerous one. Currently, emissions from mobile combustion sources are the main agents responsible for pollution, although a significant decrease is expected in Europe within the next decade. The surfaces of architectural heritage in urban environment are exposed to degradation due to the interaction with atmospheric pollutants both in gaseous and in particulate phase. Monuments located in the historic centres of large cities are subjected to typical anthropogenic emissions. The precise identification of the main substances responsible for the surface degradation phenomena, in particular leading to blackening, erosion of carbonatic matrices and disintegration, is essential for the definition of conservative intervention and maintenance strategies, as well as for the development of emission reduction policies on a larger scale.

Stones, mortars or bricks specimens have been sampled in Mlian from monuments of historical interests and analyzed by a multi-analytical approach including FT-TIR spectroscopy, atomic emission spectroscopy (ICP-OES), ion chromatography (IC) and electron microscopy coupled with energy dispersive X-ray spectroscopy (SEM-EDX). The characterization of the carbon fraction (organic carbon, OC, and elemental carbon, EC) was performed using a new approach based on a thermal protocol and on the use of CHN analysis and thermogravimetric analysis (TGA). Data acquired on the specimens have been compared with those collected by the regional environmental protection agency (ARPA Lombardia) in order to assess, also through the use of  dose-response functions, the degradation phenomenon that occur.

Recent Publications

- D. Gulotta, M. Bertoldi, S. Bortolotto, P. Fermo, A. Piazzalunga, and L. Toniolo (2013) The Angera stone: A challenging conservation issue in the polluted environment of Milan (Italy), Environ. Earth Sci., vol. 69, no. 4, pp. 1085–1094, 2013.

- P. Fermo, R. G. Turrion, M. Rosa, and A. Omegna (2014) A new approach to assess the chemical composition of powder deposits damaging the stone surfaces of historical monuments, Environ. Sci. Pollut. Res. Int.

- La Russa M.F., Fermo P., Comite V., Belfiore C.M., Barca, D., Cerioni A., De Santis M.,  Barbagallo L.F., Ricca M.,  Ruffolo S.A., The Oceanus statue of the Fontana di Trevi (Rome): The analysis of black crust as a tool to investigate the urban air pollution and its impact on the stone degradation (2017)  Sci. Total, Environ.  593-594, p. 297-309.

- Fermo, P., Cappelletti, G., Cozzi, N., Padeletti, G., Kaciulis, S., Brucale, M., Merlini, M. (2014) Hydrophobizing coatings for cultural heritage. A detailed study of resin/stone surface interaction Applied Physics A: Materials Science and Processing, 116 (1), pp. 341-348

- Pino, F., Fermo, P., La Russa, M., Ruffolo, S., Comite, V., Baghdachi, J., Pecchioni, E., Fratini, F., Cappelletti, G., (2017) Advanced mortar coatings for cultural heritage protection. Durability towards prolonged UV and outdoor exposure, Environmental Science and Pollution Research, 24 (14), pp.

- 12608-12617

Fig. 1 two images acquired on an area affected by the presence of black damaged layers

Biography (150 word limit)

Paola Fermo graduated in chemistry in 1993; researcher in Analytical Chemistry at University of Milan since 1999; since 2014 associate professor in Analytical hemistry. Principal investigator of national and international projects. She is author 103 scientific publications on international peer-review journals (Scopus and Web of Science) in the field of environmental and analytical chemistry as well as archaeometry; 20 conference proceeding; 16 book chapters; more than 200 communications in national and international congresses; invited speakers in national and international congresses.

Email: paola.fermo@unimi.it