Ecological and health Risk assessment of heavy metal in agricultural soil and surface water of Sewakht Chitral, North Pakistan

Muhammad Ishaq and Inayat ur Rehman

Institute of chemical sciences, University of Peshawar, Pakistan

Abstract:

The current study has been carried out to understand the distribution and metal accumulation in the agricultural soils and surface water of Sewakht area. A total of 54 soil samples from different agricultural fields and 38 surface water samples in Sewakht were collected and analyzed for metals like Cd, Co, Cr, Cu, Fe, Mn, Pb, Ni, Zn and Mo using atomic absorption spectrophotometer (Perkin Elmer, AAS-PEA-700).

Soil pollution has been assessed using pollution indices like contamination factor (CF), geoaccumulation index (Igeo), enrichment factor (EF), degree of contamination (CD) and potential ecological risk index (PERI). Chronic exposure to heavy metals and from surface water has also been evaluated.

CF showed “moderate contamination” (1 < CF < 3) of Cd, Co, Fe and Mo. Igeo indicated that the agricultural soils are “moderately polluted” (1 < Igeo < 2) due to Cu accumulation. EF showed “minor enrichment” (1< EF < 3) of Cd (1.56), Mo (1.46) and Fe (1.00). Similarly, CD has indicated “moderate degree of contamination” (6 < CD < 12) i.e. 9.73 at an average for all the sites. Potential ecological risk factor (PERI) signified moderate ecological risk of (61.08) for Cd and low ecological risk of Mo (30.38) followed by Co (6.34), Cr (0.66), Cu (3.53), Fe (0.67), Mn (2.05), Ni (0.34), Pb (0.58).

The potential ecological risk index (PERI) has shown very high ecological risk (RI > 380) at sampling point 7 (Sunitz Gol) and 32 (AWI) considerable ecological risk (190 < RI < 380) at sites 4 (Siwarth), 8 (Sunitz Gol) 11, 12, 16, 18 (Hassan Abad), 26 (Shoghor), 35 (AWI) and 50 (Shali) “moderate ecological risk index” (105.62) at an average for all the rest of the sites studied in Sewakht valley, district Chitral, Northern Pakistan.

Exposure to contaminated soils and water may possibly be the responsible sources for the prevailing diseases in the area. The most common diseases observed during the field visit were cardiac arrest, difficult breathing, hypertension, liver and kidney problems, stomach cancer, breast cancer, lung cancer as reported during visit by the inhabitants of the area.

Keywords. Heavy metals, soil, surface water, index analysis, potential ecological risk index, chronic exposure