Nigella sativa L. seeds biomass as a potential sorbent in sorption of lead from aqueous solutions and wastewaters.

Abdelhamid Addala1, Noureddine Belattar1, Maria Elektorowicz 2

1-Applied Biochemistry Laboratory, Department of Biochemistry, Ferhat Abbas University,      Sétif 01, 19000 Sétif, Algeria.

2- Department of Building, Civil and Environmental Engineering, Concordia University, Canada.

Abstract

Every year millions tons of biomasses issues from agriculture activities and food factories are discarded as a waste. These bulk vegetal biomasses constitute a potential and promoting material in heavy metal pollutants remediation. This first study investigated the efficiency removal of Pb(II) from solution and wastewater effluents using Nigella sativa seeds waste biomass. The experiments were performed in batch mode under various parameters such as contact time, initial Pb(II) concentration, initial pH and biosorbent dose. The biosorption data was analyzed using pseudo-first and pseudo-second-order kinetic models of Langmuir and Freundlich isotherms. The sorbent was characterized by FTIR. The optimal conditions such as equilibrium contact time for the removal of Pb(II), amount of adsorbent dose and pH were 360 min, 0.07 g and pH > 4. Under these conditions, the corresponding response for maximum biosorption yield was 90.31 mg/g. The Langmuir model and pseudo-second order kinetic fitted well to the adsorption experimental data. The successfully removing trial of lead from activated sludge waste and its decanted effluent was also confirmed under the above mentioned conditions and the sorption efficiency attain 97% and 64% respectively. Based on the results, the Nigella sativa seeds waste biomass appears as a promising biosorbent for remediation of lead pollutant from wastewaters.

Keys words: Nigella seeds meals waste; Biosorption; Biosorbent; Biomass sorbent; Lead sorption; Lead removal; Sorption isotherm.