Optimisation of catalase activity by Rhodotorula glutinis using experimental design

Ayse Ezgi Unlu, Faculty of Engineering, Department of Chemical Engineering, Ankara University, Turkey

Serpil Takac, Faculty of Engineering, Department of Chemical Engineering Ankara University, Turkey

### Abstract

Rhodotorula glutinis is a pigmented, salt tolerant yeast and also gains attention due to its oleogenic property. It has high capacity to produce antioxidant molecules such as carotenoids.1,2 However, limited research has been conducted on the synthesis of other antioxidant molecules such as catalase (CAT) enzyme. CAT is an hemeprotein that is present in animal cells, bacteria and plants and it decomposes hydrogen peroxide to water and oxygen (Eqn (1)). It is widely used in various industrial areas such as textile, food and cosmetics.3-6

2 H2O2    O2 + 2 H2O                                                             (1)

The motivation of this study is to (i) to investigate the parameters that provided the optimum conditions for high CAT activity by Rhodotorula glutinis. (ii) to search for the potential utilization of glycerol as a carbon source for high CAT activity, which is a by-product of biodiesel plants. For this aim, Central Composite Design (Design Expert 7.0.0) including 20 runs with 6 central points was performed and temperature (T,°C) (10.6-32.4°C), initial medium pH (pH) (3.99-6.0) and glycerol concentration (Gly, gL-1) (9.77-60.23) were selected as factors to be optimized for the response, CAT activity, according to the previous findings of the research group.7 The following second order model (Eqn (2)) was proposed:

CAT (U) = -4.36106 + 1.34381(pH) + 0.051575(T) + 0.028907(Gly) - 0.13551(pH)2 - 3.78071x10-4(Gly)2                                                                   (2)

The model was found to be statistically significant (R2= 0.94, R2adj = 0.92, model F value 40.95, lack of fit value 2.21). The most effective factor on CAT activity was found as temperature (p<0.0001). The response surface graphics are presented at Fig. 1. Fig. 1a was obtained when Gly was 37.03 mg mL-1. According to the figure, the highest CAT activity was obtained at high T and low pH values. Similarly, Fig. 1b showed that activity increased with increasing T however, medium values of Gly provided higher activity values, maximum at 37.5 mg mL-1. According to Fig. 1c, the highest CAT activity values were obtained at the medium values of both pH and Gly. As a result of the experiments, it was found out that determination of the maximum predicted response required a shift of the experimental region to higher temperature values.

### Fig. 1. Response surface graphics of CAT activity (a) T-pH (b) T-Gly (c)

### pH-Gly

References

- Bhosale P B, Gadre R V (2001) Production of β-carotene by a mutant of Rhodotorula glutinis. Appl. Microbiol. Biotechnol 55: 423-427.

- Yen H W, Zhang Z (2011) Effects of dissolved oxygen level on cell growth and total lipid accumulation in the cultivation of Rhodotorula glutinis. J Biosci Bioeng 112:71-74.

- Pinheiro R, Belo I, Mota M, (1997) Physiological behaviour of Saccharomyces cerevisiae under increased air and oxygen pressures. Biotechnol Lett 19:703-708.

- Havas J, Nagy G, Porjesz E, Pungor E, (1983) Molecule selective sensor for industrial use and procedure for its preparation, Google Patents.

- Dondero M, Egaña W, Tarky W, Cifuentes A, Torres J A (1993) Glucose oxidase/Catalase improves preservation of shrimp (Heterocarpus reedi). J Food Sci 58:774-779.

- Cook J N, Worsley J L (1996) Non-mammalian-derived catalase, Google Patents.

- Unlu A E, Takac S (2012) Investigation of the simultaneous production of superoxide dismutase and catalase enzymes from Rhodotorula glutinis under different culture conditions. Artif Cells Blood Substit Immobil Biotechnol 40:338-344.

Biography

Ayse Ezgi Unlu has studied about enzymes, enzymatic reactions, fermentation, protein synthesis, proteomics, enzymatic biopolymers and green solvents. The synthesis of Naproxen, a member of NSAIDs, was the subject of the master thesis using commercial lipase subjected to various pre-treatment strategies that enhanced the activity. Investigation of different parameters on the production of lipase by Candida rugosa and also proteomic analysis of the isoenzymes was another subject of interest. Two important antioxidant enzymes, catalase and superoxide dismutase production by Rhodotorula glutinis was studied comprehensively during PhD thesis. A post-doc research on the synthesis of flavonoids using green solvents was completed.

Email: aeunlu@eng.ankara.edu.tr