# Titanium Dental Implant: Macro and Micro Features

Ahmed Hindy, Ph.D., University of Babylon, IRAQ

Abstract

Background: Titanium (Ti) and titanium alloys have been used widely as the biomaterial of choice for dental implant since the early 1970s and considered as the gold standard biomaterial since it has a bulk combination of properties of biocompatibility, mechanical strength, and chemical stability. The study of dental implant structure is relevant in order to better understand the interaction of the titanium surface structure and the surrounding biological tissue. Dental implant topography refers to macro and microscopic features of the implant surface. Ti dental implant surface modification via physical, chemical and mechanical methods are recommended and applied, as well as their combination.

Objectives: this review summarizes the recent developments in the field of titanium dental implant design and surface treatment for the purpose of biocompatibility enhancement and though better osseointegration.

Materials and Methods: Using the PubMed search engine for the period time 2000-2015, the literature search was conducted to identify studies related to this topic. The most significant studies were selected based on study design, and the results were correlated. 559 studies were identified in the preliminary search, 127 studies meet the inclusion criteria in regard to the title, and 10 studies were selected from the best studies reviewed.

Results:

Macro structure Features: The most popular body shape of which is the endosseous (bladelike, cylindrical, disk-like, screw shaped, and tapered with screw shaped) implant. Shortly after the development, the use of the endosseous root shape dental implant became the gold standard. The addition of threads to the body design is allowed for an increase bone-to-implant contact potential. Thread pitch, depth and configuration or shape found to play a role in the stress distribution of an implant to the surrounding bone. Favorable and unfavorable force distribution is the key to design the dental implant macroscopic features. Tensile and shear forces are thought to be unfavorable due to an observed weakening of the bone and so, increasing compressive forces are focused on. Efforts to attain this have been made through tapering of the implant body and adjusting the thread design.

Micro structure Features: The impact of biomaterial biocompatibility and microstructure on considering features essential to achieving the osseointegration have been shown to play a role for success. Corrosion and cellular reactions were considered as the key factors in the biomaterial selection.

Surface Morphology: Based on several studies that, to gain complete growth of bone around implant surface, its irregularities should be at the level of microscale. Variation of the surface microstructure has been reported to influence the stress distribution, retention of the implants in bone and cell responses to the implant surface.

### Conclusions:

### The implants with rough surfaces have improved bone response, with bone trabeculae growing in a perpendicular direction to the implant surface. Micro-topography appear to be a factor that influences the differentiation of mesenchymal cells into osteoblasts. Cells on micro-rough Ti substrates form hydroxyapatite in a manner that is more typical of bone than do cells cultured on smooth surfaces. Thus, on micro-rough Ti surface osteoblasts create an environment conducive to new bone formation.

Key words: Macro structure, Micro structure, Titanium Dental Implant, Implant Geometry, Thread Design, Implant Biomaterials, and Surface Modification.

### Image

Recent Publications (minimum 5)

- SangHyun Lee KJ, and KyuBok Lee. Effect of thread design on the marginal bone stresses around dental implant. J Korean Acad Prosthodont 2011;49(4):6323-31.

- Chang PK  CY, Huang CC, Lu WH, Chen YC, Tsai HH. Distribution of micromotion in implants and alveolar bone with different thread profiles in immediate loading: a finite element study. Int J Oral Maxillofac Implants 2012;27(6).

- Luigi Baggi IC, Franco Maceri, Giuseppe Vairo. Stress-based performance evaluation of osseointegrated dental implants by finite-element simulation. Simulation Modelling Practice and Theory. 2008;16: 971–87.

- Manisha G Herekar VNP, Shahnawaz S Mulani, Megha Sethi, Omkar Padhye. The in fl uence of thread geometry on biomechanical load transfer to bone: A finite element analysis comparing two implant thread designs. Dental Research Journal. 2014;11(4):489-94.

- Serhat Emre Ozkir HT. Macro design effects on stress distribution around implants: A photoelastic stress analysis. Indian J Dent Res 2012;23(5):603-7.

- Serkan Dundara TT, Murat Yavuz Solmazc, Ferhan Yamand, Yusuf Atalaye, Arif Saybakf, Fatih Asutaye and Omer Cakmak. Finite element analysis of the stress distributions in peri-implant bone in modified and standard-threaded dental implants. BIOTECHNOLOGY & biotechnological equipmentwwwtandfonlinecom/loi/tbeq20. 2015.

- Balloni S  CE, Damiani F, Bistoni G, Calvitti M, Locci P, Becchetti E, Marinucci L. Effects of titanium surface roughness on mesenchymal stem cell commitment and differentiation signaling. Int J Oral Maxillofac Implants. 2009;24(4):627-35.

- y Zvi Schwartz PR, Ge Zhao, Yael Barak, Michael Tauber, Hai Yao, and Barbara D. Boyan. Effect of Micrometer-Scale Roughness of the Surface of Ti6Al4V Pedicle Screws in Vitro and in Vivo. J Bone Joint Surg 2008;90(d):2485-98

- SzmuklerMoncler S  PD, Ahossi V, Magnin G, Bernard JP. Biological properties of acid etched titanium implants: effect of sandblasting on bone anchorage. PMID: 1 4737762 [PubMed indexed for MEDLINE]. 2004;68(2):149-59.

- Rainer Wittig et.al. . 2012 V, No. 4. Direct laser writing-mediated generation of standardized topographies for dental implant surface optimization. J Laser Appl. 2012;24(4).

Biography

Ahmed Hindy has completed his PhD of Dental Biomaterials, in the field of Laser/Implant Dentistry in the year 2017 and has a Diploma in Oral Implantology from Freiberg University, Germany in 2011. He is now the director of the Department of Dental Biomaterials & Prosthodontic, Chief Director of the center of Implant Dentistry and the Head of the Laser Management & Research Unit, College of Dentistry-University of Babylon-IRAQ.

Presenting author details

Full name (As per the passport): Ahmed A H Hindy HAMMADI

University/Industry/Organization name: University of Babylon

Designation (Director/Prof/Assistant or Associate prof/Researcher/Student): Director/Dept. of Prosthodontic

Country: IRAQ

Mobile number (country code – number): 00964-780 123 2877

Email: ahhindy@gmail.com

Track number: 12

Category (Oral presentation/ Poster presentation/Young researcher forum/E-poster): Oral Presentation

Research interest: Implant Dentistry

Your profile page URL in your department:

Year of birth: 1969

Linked In/Facebook/twitter account: www.linkedin.com/in/ahmed-hindy-77034828/

Note: Please fill up the Red mark fields so that it would be easier for us to cite your details in the conference particulars. (Please do not forget to fix your photo and Logo above the title)