Kinetics of coherently synchronized peroxidase reaction of ethyl alcohol by a green oxidizing agent - hydrogen peroxide

S.Aghamammadova1, U.Nasirova1, I.Nagieva2, L.Gasanova1, T.Nagiev1,2

1Nagiev Institute of Catalysis and Inorganic Chemistry, National Academy of Sciences of Azerbaijan, 113 H.Javid av., az 1143, Baku, Azerbaijan

2Baku State University, 23 Z.Khalilov, az 1148, Baku, Azerbaijan

Kinetics in the model system of reaction of ethyl alcohol conversion to acetaldehyde peroxidase by hydrogen peroxide in the presence of an iron porphyrin-containing biomimetic catalyst were studied. Application of such oxidation system, as shown by experimental results, meets the basic requirements of the Green Chemistry concept.

The process was carried out in the gas phase at the temperature of 180°C and atmospheric pressure on  per-FTPhPFe3+OH/Al2O3 heterogeneous biomimetic catalyst using 20% ​​hydrogen peroxide. As a result of the process acetaldehyde was obtained with the yield of 93.5% and selectivity of 98% (taking into account the loss).

Through the experimental study of kinetic laws of ethyl alcohol peroxidase reaction, coherently synchronized nature of the mechanism of ethyl alcohol conversion to acetaldehyde by hydrogen peroxide on the surface of the biomimetic was identified. Thereby there was a need for a special approach to the kinetic modeling of this reaction. The mechanism presented demonstrates that the following two interconnected and interacting reactions proceed in this system: the reaction of Н2О2 decomposition of and the reaction of peroxidase oxidation of ethyl alcohol, according to [1-3].

The condition for coherence between these synchronously proceeding reactions is their quantitative characteristic determined by determinant equation [1]:

D=νrA1rAcc+rA2rAcc-1

where the D factor in the area of chemical conjugation is D = 0 ÷ ν.

rA1и rA2- rates of actor (Н2О2) consumption for the formation of final products in the primary (Catalase) and secondary (Peroxidase) reactions, respectively.

ν is stoichiometric coefficient of the actor.

The use of this equation in the kinetic modeling makes it possible to obtain more adequate values ​​for kinetic parameters, which are within the range of parameters characteristic of enzymatic reactions. This is facilitated by the fact that the determinant equation allows to consider the rates of both simultaneously occurring coherently synchronized reactions within the kinetic model.

References

- T. M. Nagiev, Coherent Synchronized Oxidation Reactions by Hydrogen Peroxide. Amsterdam: Elsevier. 2007. 325 р

- T. M. Nagiev, Interaction of Synchronous Reactions in Chemistry and Biology. Baku: Elm. 2001. 404 р. [in Russian].

- Nagiev T., Gasanova L., Nagieva I., Nasirova U., Kinetics and Mechanism of Coherent Synchronized Ethylene Monooxidation by Hydrogen Peroxide on a Biomimetic Catalyst, per-FTPhPFe3+OH/Al2O3 // Journal of Material Science and Engineering, 2012, Vol. 2, No. 4, pp. 306-312.

Biography

Tofik Nagiev is a Vice-president of Azerbaijan National Academy of Sciences, Director of Research Center of “Azerbaijan National Encyclopedia” and Department chief of Nagiev Institute of Catalysis and inorganic chemistry of ANAS. The Professor of the department of the physical and colloid chemistry of Baku State University.

He is author of the monography “Coherent Synchronized Oxidation Reactions by Hydrogen Peroxide”, Amsterdam: “Elsevier”, p. 325, 2007.

Presenting author details
Full name: Tofik Nagiev

Contact number: +99412 5384707

Category: Poster