Photo					                            University/organization Logo

Title of speech: Automated Open-Access Liquid Chromatography High Resolution Mass Spectrometry to Support Medicinal Chemistry Projects

------------------------------------------------------------------------------------------------

Name: Alberto Fontana

University/ Organization, Country: Janssen Research & Development, Spain

Alberto Fontanaa, Laura Iturrinoa, Corens, Davidb, Antonio L. Cregoc

a Analytical Sciences, Janssen Research & Development, a Division of Janssen-Cilag S.A., C/Jarama 75A, 45007 Toledo, Spain.

b Analytical Sciences, Janssen Research & Development, a Division of Janssen Pharmaceutica NV, Turnhoutseweg 30, B-2340 Beerse, Belgium.

c Department of Analytical Chemistry, Physical Chemistry, and Chemical Engineering, Faculty of Biology, Environmental Sciences and Chemistry, University of Alcalá, Ctra. Madrid-Barcelona, Km. 33.600, 28871 Alcalá de Henares, Madrid, Spain.

Abstract:

The need of a continuous productivity increase in the pharmaceutical industry motivated the development of new Open-Access tools on many analytical techniques [1-5]. On LC, these platforms have been commonly used with low-resolution MS detectors while they have been reported with very limited automation on high-resolution MS spectrometers [6]. Typically, samples with concentration ranges around 0.1 mg/mL are prepared by medicinal chemists for reaction monitoring and for UV purity assessment. Together with the evolution of the instrumentation, new software packages to automatically process HRMS data have appeared [7, 8]. These systems are typically run by specialists and developed for biological samples with compounds in concentrations of µg/mL range.

In this work, the development of a Fully Automated Open-Access methodology on a UHPLC-DAD/ESI-QTOF system is reported, being applied to medicinal chemistry discovery projects from Janssen R&D. With the first MS conditions applied, accuracies < ±1 mDa were not achieved in 20-30% of the cases in ESI+ due to oversaturation. After optimization of MS conditions, a new ESI+ method was implemented. This method has been used by chemists for more than 2.000 compounds from structurally diverse chemical series with a success >98% in exact mass determinations. For the other 2 % of the samples, specific conditions are used.

Upon training, chemists log-in samples and interpret the results by themselves in 90-95 % of the cases. To our knowledge, this the first time that this Full Automation has been achieved for an Open-Access LC-DAD-HRMS platform developed for end-users in a pharmaceutical environment.

References:

[1] Leo G.C., Krikava A., Caldwell G.W., Application of flow NMR to an open-access pharmaceutical environment. Anal. Chem. 2003, 75, 1954-1957.

[2] Mallis, L.M; Sarkahian, A.; Kulishoff, J.M.; Watts, W.L., Open-access liquid chromatography mass spectrometry in a drug discovery environment, J. Mass Spectrom., 2002, 37, 889-896.

[3] Coddington, A., Van Antwerp, J., Ramjit, H., Critical considerations for high-reliability open access LC/MS, J. Liquid Chrom. Related Technol., 2003, 26, 2839–2859.

[4] Bu, X., Yang, J., Gong, X., Welch, C.J., Evaluation of a compact mass spectrometer for routine support of pharmaceutical chemistry, J. Pharm. Biomed. Anal., 2014, 94, 139–144.

[5] Gao, J., Ceglia, S.S., Jones, M.D., Sime one, J., Van Antwerp, J., Zhang, L-K., Ross III, C.W., Helmy, R., A novel compact mass detection platform for the open access (OA) environment in drug discovery and early development, J. Pharm. Biomed. Anal., 2016, 122, 1-8.

[6] Thomas, S.R, Gerhard, U., Open-access high-resolution mass spectrometry in early drug discovery, J. Mass Spectrom., 2004; 39, 942-948.

[7] Taibon, J., Sturm, S., Seger, C., Parth, M., Strasser, H., Stuppner, H., Development of a fast and selective UHPLC-DAD-QTOF-MS/MS method for the qualitative and quantitative assessment of destruxin profiles, Anal Bioanal Chem., 406, 29, (2014), 7623-7632.

[8] Llorca, M., Lucas, D., Ferrando-Climent, L., Badia-Fabregat, M., Cruz-Morató, C., Barceló, D., Rodríguez-Mozaz, S., Suspect screening of emerging pollutants and their major transformation products in wastewaters treated with fungi by liquid chromatography coupled to a high resolution mass spectrometry, J. Chromatogr. A, 2016, 1439, 124-136.

Biography:

Alberto Fontana completed his Master of Science in Chemistry from University Autónoma of Madrid. He joined Janssen Research & Development (Spain) in 1998. Since 2001, he works as Scientist in the Analytical Sciences Team. He is focused on analytical LCMS, GCMS and on preparative LC, developing new methodology for Medicinal Chemistry groups, mainly within Neuroscience area. He is currently doing his PhD studies at University of Alcalá de Henares, Madrid. He is the main author of 1 publication and co-author of another 5 papers. He has presented several posters with his research work on LCMS at different international congresses.

Mail Id												afontana@its.jnj.com