A Study on chemical finishing of Ti6Al4V processed by EBM

C. Pirozzi1, F. Scherillo2S. Franchitti1,  R. Borrelli1,

1CIRA - Italian Aerospace Research Centre, Via Maiorise 1,81043 Capua, Italy

e-mail: c.pirozzi@cira.it

2DICMAPI – University of Napoles  ’Federico II’, Piazzale Vincenzo Tecchio, 80, 80125 Napoli

Electron Beam Melting (EBM) is one of a few AM technologies capable of making full-density functional metallic parts realized from raw materials in the form of powders [1]. EBM utilizes a high-energy electron beam, as a moving heat source, to melt and solidify, by rapid self-cooling, metal powder and produce parts in a layer-building fashion [2], [3].In particular, the ability of direct fabrications of metallic parts can accelerate product designs and developments in a wide range of metallic-part applications, especially for complex components, e.g., fine network structures ([4], [5]), internal cavities and channels, which are difficult to make by conventional manufacturing means. Anyway many technical aspects concerning the quality of EBM produced components are still industrial open items and studies need to be carried out, expecially the high level of the surface roughness. In according to the industrial needs in  this work it have been studied a chemical finishing in order to improve the surface roughness of Ti6Al4V components fabricated by EBM production technology.

References

- A. Uriondo, M. Esperon-Miguez, and S. Perinpanayagam, The Presentand Future of Additive Manufacturing in the Aerospace Sector: A Review of Important Aspects, Proc. IMechE Part G: J. Aerosp. Eng., 2015, doi:10.1177/0954410014568797§

- D. X. Gong, T. Anderson and K. Chou, Review On Powder-Based Electron Beam Additive Manufacturing Technology, Manufacturing Rev., 2014, 1(2), p 1-12

- C. Koerner, Additive Manufacturing Of Metallic Components By Selective Electron Beam Melting — A Review. Internat. Mat. Rev., 2016, 61(5), p 361-377

- O. Cansizoglu, D. Cormier, O. Harrysson, H. West, and T. Mahale, An Evaluation of Non-Stochastic Lattice Structures Fabricated Via Electron Beam Melting, Edward P. Fitts Department of Industrial and Systems Engineering Raleigh, NC 27695-7906.

- W. Van Grunsven, E. Hernandez-Nava, G. C. Reilly and R. Goodall, Fabrication and Mechanical Characterisation of Titanium Lattices with Graded Porosity, Metals 2014, 4, 401-409.

Biography

Carmine Pirozzi Graduated with honors in Industrial Engineering in 2004 with a PhD. in Systems and Technology of Production. He is researcher in additive manufacturing since 2011, involved in projects as HYPROB, TIMA-RITAM, ANGELA, SAT-AM as material science and additive manufacturing engineer specialist. He does coperate with several research centers, universities and firms being authors of more than 10 scientific articles on specialised Journals and conferences.

Presenting author details 
Full name: Carmine Pirozzi
Contact number: +393389047923
Twitter account:
Linked In account:

Session name/ number: Material science and engineering / Surface Engineering

Category: Oral presentation