MINERALOGICAL COMPARATIVE EVALUTION OF LIGINTE AND BITUMINOUS COALS FROM IHIOMA AND ENUGU RESPECTIVELY

ABSTRACT

A sample of Ihioma coal in Orlu L.G.A of Imo State and 9th mile coal of Enugu State which are lignite and bituminous coal respectively was analyzed to finding their chemical composition using XRD-EDS, proximate and ultimate analysis to determine their moisture content ash content, volatile matter, and mineral composition, where Enugu coal recorded 3.5% moisture content,4.5% Ash content,32.0% volatile matter and 60% fixed carbon with high values in sodium 7.2%,aluminum 32.0% and silicon 61.0% in its mineral composition. A two way ANOVA was used to test the variations of data results when F- test at a level of 5% of probability was applied, the averages followed by the same letter do not differ statistically between themselves at 1% level implying that there is 99% confidence in results gotten from this coal samples. This Ihioma coal sample based on it’s diffraction pattern showed the contents to be SiO2, 8.23%, Al2O3, 14.75%, Fe2O3, 70.81%, CaO, 0.06%, Na2O, 0.02%, K2O, 0.04%, TiO2, 0.01%. The mineralogical analysis showed the presence of hematite, quartz, Albite and trace metals of metakaolin. The SEM-EDS also showed the distribution of these minerals within the coal body.