Application of Shrinking Core Model to Supercritical CO2

# Extraction of Algerian Argan (Argania Spinosa L) Oil Seeds

N Chadi, MSc, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

I Kadri, MSc, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

I Haloui, PhD, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

A Zermane, PhD, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

N Outili, PhD, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

A-H Meniai*, PhD, Laboratoire de l’Ingénierie des Procédés de l’Environnement (LIPE), Université Constantine 3

### Abstract (300 word limit)

Generally the mass transfer mechanism taking place at high pressure during the extraction of complexes compounds like essential oils is not fully explained. Therefore a number of different models are described and reported in the literature basing on mass balances over fixed beds of the considered solid supports.

The aim of the present study is to outline the bases of one of these models, known as the “shrinking core” model and which is used for the modeling of a supercritical carbon dioxide extraction of essential oil from various local plants like Argan and Fennel.

Although its complexity the shrinking core model showed a great reliability. This was achieved by means of reasonable and simplifying assumptions which enabled the resolution of a set of linked differential equations obtained by writing appropriate solute mass balances over the plant particles.

A computing code was developed in Mathcad Ver.14 for the resolution of the different involved differential equations along with appropriate initial and boundary conditions, leading to solute concentration profiles in the plant solid particles assumed to be spherical. The integration of these concentration profiles over the whole height of the particle bed led to the oil extraction yield.

Numerically the obtained results were in a quite good agreement with the experimental results and this for Argan and Fennel plants, encouraging the further development of this model.

### Image

Recent Publications (minimum 5)

- Machmudah S, Sulaswatty A, Sasaki M, Goto M, Hirose T (2006) ‘Supercritical CO2 extraction of nutmeg oil: Experiments and modeling’, J. of Supercritical Fluids 39:30–39.

- Doker O , Salgin U, Yildiz N , Aydog˘mus M, Calimli A (2010), ‘Extraction of sesame seed oil using supercritical CO2 and mathematical modeling’, Journal of Food Engineering 97:360–366.

- Zermane A, Meniai A-H, Larkeche O , Barth D (2012), ‘Extraction and Modeling of Algerian Rosemary essential oil  using supercritical CO2: Effect of pressure and temperature’, Energy Procedia 18:1038 – 1046.

- Haloui I, Meniai A-H (2017), ‘Supercritical CO2 extraction of essential oil from Algerian Argan (Argania spinosa L.) seeds and yield optimization’, International Journal of Hydrogen Energy, vol. 42, issue 17.

- Belbaki A , Louaer W, Meniai A-H (2017), ‘Supercritical CO2 extraction of oil from Crushed Algerian olives’, Journal of Supercritical Fluids, vol. 130.

- Zermane A, Larkeche O, Meniai A.-H., Crampon C, Badens E (2016) ‘Optimization of Algerian rosemary essential oil extraction yield by supercritical CO2 using response surface methodology’, Comptes Rendus Chimie, 19:538-543.

- Djerafi R, Masmoudi Y, Crampon C, Meniai A-H, Badens E (2015), ‘Supercritical Anti-Solvent Precipitation of Ethyl Cellulose’, The Journal of Supercritical Fluids, Volume 105:92-98.

Biography (150 word limit)

Abdeslam-Hassen Meniai is a full professor in Chemical Engineering since 1998. He has authored more than 150 papers and 250 conference papers in this field, mainly concerning Molecular simulation, Liquid-liquid extraction, Thermodynamics of phase equilibria and Optimization of chemical processes. He has been an elected member of the Institution of Chemical Engineers (IChemE) as well as the Fluid Separation Group (Great Britain). He has also been a member of a great number of international and national research projects, all based on chemical engineering. One of his main research achievements is the development of an original method for the three dimensional simulation of liquid mixtures coupling molecular modeling and the Monte Carlo simulation. He is one of the pioneers of the use of Molecular Graphics Systems (MGS). He has made use of this system to develop a criteria fully based on fundamental calculations of molecular interaction energies for the molecular design and selection of solvents for liquid-liquid extraction.

Currently he is considering the improvement of existing thermodynamic models and also the calculation of phase equilibria at high pressure, particularly for applications to supercritical fluid extraction from local natural plants.

Email: meniai@yahoo.fr

Notes/Comments: