Title: An Experimental Study To Measure The Concentration Of Nanofluid By Using Refractive Index

Name: NG Kwan

RMIT Unversity, Melbourne, Australia

Name: CHAU Man Hei

School for Higher and Professional Education, Vocational Training Council, Hong Kong

The concentration plays a decisive role in nanofluids. Most thermo-physical properties changes of nanofluids, such as thermal conductivity, thermal diffusivity, viscosity and convective heat transfer coefficient, all depend on the concentration of nanofluids. However, there are no recognized effective methods for measuring the concentration of nanofluids after the preparation until present.  This matter will hinder nanofluid quality ensurance and the real time monitoring on the performance of nanofluids during the operation. Thus, the purpose of this research is to experimentally measure the concentration of Titanium Oxide (TiO2) nanofluids by refractive index minimum deviation method. TiO2 nanopowder of 21nm and with selected distilled water to the experiment. The two-step method was adopted to prepare nanafluids. The experiments were conducted by measuring the laser beam displacement in room temperture 22oC and concentration of 10 TiO2 nanofluids samples with practical mass fraction of nanopoweder ranging from 0.001% to 0.04%. The refractive index of the samples were then calculated based on Snell’s law. According to the calculation, the correlation between refractive index and concentration of nanofluids will be interpreted. Results of the experiment show that the refractive index value roughtly linearly increased from 1.341 to 1.395 by increase in the mass fraction concentration. Results of this works will be very helpful for further development of the precise methods for measurement of nanofluids concentration.

Biography

NG Kwan presently attending a Bachelor of Engineering Degree in Mechanical Engineering  (Honours) in RMIT University. He obtained the RMIT School of Engineering Award in 2018. He received a Higher Diploma in Mechanical Engineering (Distinction) in Hong Kong Institute of Vocational Education in 2017. He have more than 16 years experience in inspection, test and examination of structure, motor, plant and mechinery accordance with the currently statutory requirements. He also has more than 10-year experience in project management. After completing the Bachelor Degree program, he is interested in continuing his study on the technology and application of nanofluids.

CHAU Man Hei is the Programme Coordinator of School for Higher and Professional Education of VTC, Chactered Professional Engineer of The Institution of Engineers Australia and Committee Member of IEEE (Electronics Packaging Society). He is responsible for the programme development and management of BEng in Mechanical Engineering of RMIT University.

Presenting author details 
Full name: NG Kwan
Contact number: +852 61056114
Twitter account: Napoleon Ng@k_npln
Linked In account: https://www.linkedin.com/in/kwan-ng-23b408149

Session name/ number: Nano Photonics, Nano Imaging, Spectroscopy & Plasmonic Device / Track 8
Category: Oral presentation