Fabrication of few-layer and functional MoS2 and WS2 effectively and their applications for ultrafast molecule separation.

Yuyu Su, Dan Liu, Lifeng Wang, Weiwei Lei.

Institute for Frontier Materials, Deakin University, Waurn Ponds Campus, Locked Bag 20000, Geelong, Victoria 3220, Australia.

Abstract

Molybdenum sulphide (MoS2) and tungsten sulphide (WS2) nanosheets, two typical kinds of two-dimensional transition metal sulphide, have attached much attention due to their excellent properties, such as direct bandgaps, electron mobility in plane, catalytic activity and chemical stabilities.1 However, fabrication of few-layer MoS2 and WS2 with high yields effectively is still challenge and their applications are limited practically2 Therefore, the purpose of this study is to prepare few-layer and functional MoS2 and WS2 effectively and explore their applications in molecule separation. It has been reported that mechanichemical exfoliation was a good method to produce 2D materials with high yields effectively.3 In this study, through mechanichemical exfoliation, few-layer and functional MoS2 and WS2 were prepared in 2 hours with high yields (90 %). Size and thickness of few-layer MoS2 and WS2 are approximately 200 nm and 4 nm, respectively. Both nanosheets show good dispensability in water with concentrations (5 mg/ml) and high stability for 3 months. Moreover, as-prepared MoS2 and WS2 based membranes fabricated by vacuum filtration, exhibit ultrafast molecule separation. This work provides a new approach to produce few-layer MoS2 and WS2 in a facile and effective method and enhances their performance in ultrafast molecule separation.

References

1.	M. Chhowalla, Z. Liu and H. Zhang, Chemical Society Reviews, 2015, 44, 2584-2586.

2.	J. N. Coleman, M. Lotya, A. O’Neill, S. D. Bergin, P. J. King, U. Khan, K. Young, A. Gaucher, S. De and R. J. Smith, Science, 2011, 331, 568-571.

3.	W. Lei, V. N. Mochalin, D. Liu, S. Qin, Y. Gogotsi and Y. Chen, Nature communications, 2015, 6, 8849.