CVD Growth Frontiers of Industrial Graphene Materials

Zhongfan Liu, Beijing Graphene Institute (BGI) &

Peking University, Beijing 100871, China

Abstract (300 word limit)

Chemical vapor deposition (CVD) is the most popular technique for industrial level production of high quality graphene materials. Great progress has been achieved since the first approach of catalytic CVD growth of graphene films in 2009. At the current stage, mass production of CVD graphene films toward graphene industry has been realized up to a level of millions of m2 per year. For single crystal graphene wafers, it also reaches tens of thousands of wafers per year with a size of 4 to 6 inches. Current research emphasis on CVD graphene materials is laid on three different directions, high-quality graphene films and wafers for general purpose; novel graphene materials with hierarchical structures or unique growth substrates; purpose-oriented graphene materials. The challenges for the CVD growth of general purpose graphene films and wafers include domain size, layer control, defect density, wrinkles, intrinsic surface contamination, uniformity, transferability, scalability and cost issue. Much efforts are also made on the direct growth of graphene on nonmetal target substrates and it is still no way to compete with catalytic CVD graphene on metal in quality. One of the most representative examples of novel graphene materials is the graphene glass fibers developed in Beijing Graphene Institute (BGI). We have succeeded in directly growing graphene on commercially widely used glass fibers with well controlled layers and qualities. Such kinds of graphene glass fibers exhibited excellent electrical conductivities with a sheet resistance ranging from kilo-ohms down to ohms level. It has been used for deicing applications with extremely high electrothermal conversion efficiency up to 94%. At BGI, we are also working for the purpose-oriented graphene materials and even customized graphene materials depending on the special needs and requests from customers. There is actually a great space in this area, which is particularly important before graphene films and wafers find their practical application market in next ten years.

Recent Publications (minimum 5)

- L Lin, HL Peng*, ZF Liu*, Synthesis challenges for graphene industry, Nature Mater. 18(2019), 520-529.

- K Chen#, X Zhou#, X Cheng#, RX Qiao, Y Cheng, C Liu, YD Xie, WT Yu, FR Yao, ZP Sun, F Wang, KH Liu*, ZF Liu*, Graphene photonic crystal fibre with strong and tunable light-matter interaction, Nature Photonics 13 (2019), 754-759.

- LZ Sun, ZH Wang, YC Wang, L Zhao, YL Li, BH Chen, SH Huang, SS Zhang, WD Wang, D Pei, HW Fang, S Zhong, HY Liu, JC Zhang, LM Tong, YL Chen, ZY Li, MH Rummeli, KS Novoselov, HL Peng*, L Lin*, ZF Liu*, Hetero-site nucleation for growing twisted bilayer graphene with a wide range of twist angles, Nat. Commun. 12 (2021), 2391.

- LZ Sun, GW Yuan, LB Gao*, J Yang, M Chhowalla*, MH Gharahcheshmeh, KK Gleason*, YS Choi, BH Hong*, ZF Liu*, Chemical Vapour Deposition, Nat. Rev. Methods Primers 5 (2021), 5.

- H Yuan, H Zhang, KW Huang, Y Cheng, K Wang, ST Cheng, WJ Li, J Jiang, JL Li, C Tu, XB Wang*, Y Qi*, ZF Liu*, Dual-Emitter Graphene Glass Fiber Fabric for Radiant Heating, ACS Nano, https://doi.org/10.1021/acsnano.1c09269.

Paste your Logo here

Biography (150 word limit)

Zhongfan Liu completed his PhD from University of Tokyo in 1990 and postdoctoral study from the same university and Institute for Molecular Science (IMS), Japan. His current research interests include the CVD growth, mass production and equipment manufacturing, and unique applications of graphene. He is the founding Director of Beijing Graphene Institute (BGI) and a BOYA chair professor of Peking University. He is the member of Chinese Academy of Sciences and the fellow of TWAS.

Email: zfliu@pku.edu.cn

Notes/Comments:

Presenting author details 
Full name: Zhongfan Liu
Contact number:
Twitter account:
Linked In account:

Session name/ number: 
Category: (Oral presentation/ Poster presentation)