Co-product utilization-the case of soybeans

J. K. Sekhon, Drexel University Department of Culinary Arts and Food Science, Philadelphia PA, USA

T. Wang, Iowa State University Department of Food Science and Human Nutrition, Ames IA USA and Center for Crops Utilization Research, Ames IA USA

S. Jung, California Polytechnic State University, San Luis Obispo, Department of food science and nutrition, CA, USA

K. Rosentrater, Iowa State University Department of Agricultural and Biosystems Engineering; Department of Food Science and Human Nutrition, and Center for Crops Utilization Research, Ames IA USA

### Abstract

Statement of the Problem: Soybean production accounts for

~90% of oilseeds production in the U.S. Oil from soybeans is conventionally extracted mechanically using screw press or by using organic solvent. While former process denatures the proteins, the later has potential health and environmental hazards. The drawbacks of these processes can be overcome by sustainable use of enzymes, which allows recovery of both oil and proteins. Enzyme assisted aqueous extraction process (EAEP) uses water as extracting medium and can achieve ~97% oil recovery from soybeans. However, one limitation of the EAEP process is production of surplus amounts of skim and insoluble fiber.

Biography

Jasreen is an assistant professor in the department of Culinary Arts and Food Science at Drexel University. Her research addresses utilization of co-products from the food industry, with specific focus on developing processes to utilize these co-products, study the effect of processing on the quality of the product and determining nutritional value of the developed product.

Email: jks333@drexel.edu

The purpose of this study was to determine how to best utilize

co-products, skim and insoluble fiber, produced from the EAEP of soybeans. Methodology: The effect of addition of skim and insoluble fiber on ethanol production, oil recovery and quality of dried distillers grains (DDG) in corn fermentation was investigated. Enzyme cocktail (fiber hydrolyzing enzymes and/or protease) and surfactant were used to maximize efficiency of the process. Findings: Addition of soy co-products to corn fermentation increased ethanol yield and decreased fermentation time by 38 h compared to corn-only fermentation. Maximum oil recovery was achieved from [corn only] and [corn + insoluble fiber] slurries when pectinase, cellulase and Fermgen were added to corn-soy slurry during fermentation and when Tween80 was used as a surfactant. Significance: This research has demonstrated potential of soy co-products from EAEP of soybeans in maximizing ethanol and oil recovery from corn fermentation.

Recent Publications (minimum 5)*

- Sekhon, J. K., Maness, N. O., & Jones, C. L. (2016). Effect of compressed propane extraction on storage stability of dried cilantro (Coriandrum sativum L.). Journal of Food Engineering, 178, 159-169

- Sekhon, J. K., Maurer, D., Rosentrater, K. A., Wang, T., & Jung, S. (2015). Effect of pretreatment of soy insoluble fiber and SSCF with Saccharomyces cerevisiae and Escherichia coli KO11 on ethanol production in an integrated corn-soy biorefinery. Paper presented at ASABE Annual International Meeting, New Orleans, Louisiana, 1-14

- Sekhon, J. K., Wang, T., Jung, S., Rosentrater, K. A., & Johnson, L. A. (2015). Effect of co-products of enzyme-assisted aqueous extraction of soybeans on ethanol production in dry-grind corn fermentation. Bioresource Technology, 192, 451-460

- Sekhon, J. K., Maness, N. O., & Jones, C. L. (2015). Effect of preprocessing and compressed propane extraction on quality of cilantro (Coriandrum sativum L.). Food Chemistry, 175, 322-328

- Sekhon, J. K., Maness, N. O., & Jones, C. L. (2009). Improving shelf life and quality of dehydrated cilantro by partial propane extraction. Paper presented at ASABE Annual International Meeting, Reno, Nevada, 1-8