Title: Combining Pulsed Light and Shaking System to Maximize the Degradation of B-Aflatoxins While Improving Physicochemical Properties of Treated Peanuts

Name: Manal O Abuagela1, Basheer M Iqdiam2, Eman Gusbi1, Abdalnasser M Ghraiwi1, Nabil Enattah1

- University of Tripoli. Alfornag street, Tripoli, Libya.

- Food science and Human Nutrition department, University of Florida. Gainesville, FL 32603, USA

Pulsed light (PL) is a new technique that showed promising results in the degradation of B-AFTs (B1 and B2) in peanuts. However, long periods of exposure to PL to achieve AFTs complete reduction affected the uniformity of the peanuts’ color, and produced burned spots on the surface of the treated peanuts. This study was conducted to enhance the effect of PL on AFTs and reduce the change in the physicochemical properties of the peanuts after pulsed light treatment. The experiment combined PL with a newly invented shaking system to shake peanuts during PL treatment. Two treatment distances were used (7 and 10 cm) from the PL strobe with different exposing times. B-AFTs (B1 and B2) contents were determined after the PL treatment by using ELISA and LC-MS/MS. Results concluded that PL treatment for 240s at 7 cm distance from the UV lamb degraded B- AFTs in peanut samples by 82.09% (±1.03), while the combination of PL and shaking system achieved 99.47% (±0.24) B-AFTs reduction. Statistically, significant differences (p > 0.05) were found between the two methods in the AFTs reduction percentage, peroxide value, and the color parameters. The results indicated that the combination of PL and the shaking system is an affective technique to degrade B-AFTs levels in peanuts. In addition, shaking improved the PL photochemical and photo-thermal effects, also homogenized the color of the treated kernels. At an industrial scale, combining PL and the shaking system could be an economical treatment provide commercially desired peanuts free of AFTs.

Biography

Manal Othman Abuagela, born in Tripoli Libya, received her bachelor’s and master’s degrees from the Department of Food Science, College of Agriculture, University of Tripoli in 1993 and 2006, respectively. After her bachelor’s degree, she became a chemistry teacher. After her master’s degree, Manal worked as a professor at Al-Zaytoonah University since 2006–2013. Manal started her Ph.D. studies at Department of Food Science and Human Nutrition at the University of Florida in 2013 under Dr. Wade Yang, Dr. Paul Sarnoski, Dr. Lowie Gu, and Dr. Matthew Smiths’ supervision. She graduated in 2017 and moved to Libya to continue in her job as a professor at university of Tripoli. She is teaching several topics including Food processing, Food safety, Food microbiology, Food quality control, and Human nutrition. She is also monitoring undergraduate students researches. Since then, Manal has published her work in several scientific Journals.

Presenting author details 
Full name: Manal Othman Abuagela
Contact number: 00218917544698
Linked In account: Manal Abuagela

Session name/ number: Novel Processing and Packaging Technologies

Category: (Oral presentation/ Poster presentation)