Title: Assessment of the effects of water harvesting technology on downstream water availability using SWAT model, Case of Alaba Special Woreda, Ethiopia

Name: Ayalkibet M. Seka, Adane A. Awass, Assefa M. Melesse

University of China Acadamy of Science, China

There are many water-related human interventions that modify the natural hydrological systems of watersheds. Rainwater harvesting (RWH) is such an intervention that involves harnessing of water. Water harvesting used in upstream prevents surface runoff to downstream impacting biodiversity and ecosystems. The main objective of the study is to assess the effects of water harvesting technologies on downstream water availability in Alaba District, Ethiopia. The Soil and Water
Assessment Tool (SWAT) model, cost-benefit ratio, and optimal control approach (OCA) were used to analyze the hydrological, socioeconomic impact and trade-offs on water availability of the community, respectively. The downstream impact of increasing water consumption in the upstream rainfed areas of the Bilate and Shala watershed is simulated using the semi-distributed SWAT model. The two land use scenarios tested at sub-basin levels for the conventional land use represent the current land use practice (Agri-CON) and infield rainwater harvesting (IRWH) for improving soil water availability through RWH land use scenario. The simulated water balance results showed that the highest peak mean monthly direct flow was obtained from Agri-CON land use (12.71 m3/ha), followed by Agri-IRWH land use (11.5 m3/ha). The Agri-IRWH scenario reduced direct flow by 10 % compared to Agri-CON and more groundwater flow contributed by Agri-IRWH (190 m3/ha) than Agri-CON (125 m3/ha). The overall result suggests that the water yield of the
area may not be negatively affected by the Agri-IRWH land use scenario. The technology in the district benefited positively having an average cost-benefit ratio of 4.2. Storage tanks, series of check dams, and gravel filled dams are alternative solutions for water harvesting.

Keywords: Water harvesting  SWAT  Land use scenario  Water availability  Ethiopia

Biography

Ayalkibet has completed his MS.c at the age of 24 years from Arba Minch University, Ethiopia and currently PhD research fellow student at Chinese Academy of Science at institute of Remote Sensing and Digital Earth. He is the Associate researcher of water resorces research center. He has published more than 3 papers in reputed journals and has been serving as an editorial board member of repute.

Presenting author details 
Full name: Ayalkibet Mekonnen Seka
Contact number: +86-18810505351
Twitter account: aabmdb.seka4@mail.ucas.ac.cn 
Linked In account: aabmdb.seka4@mail.ucas.ac.cn

Session name/ number:
Category: (Oral presentation/ Poster presentation)