Solubility assessment of canarium schweinfurthii (parsley) oil in supercritical carbon dioxide

Jibrin Mohammed Danlami*, Mohammed Kabir Abdullahi, Musa Hassan Abdulkadir and Jibrin Waziri

Department of Chemical Engineering, Kaduna Polytechnic, Kaduna, Nigeria

{Email: jibrin349@yahoo.com, +2347066004416}

Abstract

In this paper, the solubilities of canarium schweinfurthii oil were experimentally determined and correlated with the semi-empirical models. In solubility determination, the dynamic method was adopted and the solubilities under the pressures and temperatures of 20.7 – 34.5 MPa and 313 – 353 K were measured. The semi-empirical models containing Del Valle Aguilera (VA) model, Chrastil model, Kumar and Johnston and Mendez-Santiago and Teja (MST) models were used to correlate the solubilities of these oils and the correlation results show a good consistency with the measured solubilities. Over the temperature and pressure ranges studied, solubilities from these models had the following average absolute relative deviations from experimental data: VA, Chrastil, Kj and MST models with an average absolute relative deviation percent (AARD%) of 0.011%, 0.014%, 0.04% and 9.98% respectively. Thus, the VA, Chrastil and Kj models correlated the canarium schweinfirthii oil solubility data with the lowest AARD%.

Keywords: Solubility, Supercritical carbon dioxide, Del Valle Aguilera, Chrastil Model, Correlation, Kumar and Johnston, Mendez-Santiago and Teja