Environmental Geochemical Assessment of Ogbagha River Sediments in Okpella Area of Edo State, Nigeria

1Obaje, S. O and 1Adelowo S. A.

1*Department of Earth Sciences, Faculty of Science, Adekunle Ajasin University, P.M.B. 001, Akungba-Akoko, Ondo State, Nigeria

*Presenter Contact Details (Email, Mobile Number) { Email: solomon.obaje@aaua.edu.ng, isolobaje1@gmail.com  +234701389208}

ABSTRACT

The main aim of this paper is the environmental geochemical assessment of Ogbagha River sediments to ascertain the level of their pollution. Major and trace elements in the samples were analysed using Epsilon-5 X-ray fluorescence model (pAnalytical). The study area lies between latitude 7°14’N and 7°22’N and longitude 6°15’E and 6°23’E in Okpella area of Edo State, Nigeria. Six composite samples were used. SiO2 values range from 71.4-81.61 wt. % with mean of 75.71 wt %, while Al2O3 values range from 7.01-12.97 wt. % with mean values of 11.56 wt. %. The other major oxides are below 7.0 wt. %. SiO2 and Al2O3 wt. % values are indicative of felsic source rock. The mean concentration values of the following elements: Zr (1,830.55 ppm), Ba (1,416 ppm) and Cu (77.17 ppm) are higher than their crustal baseline values. Conversely, Sr (295 ppm), In (106 ppm), As (14.83 ppm), Cr (8.53 ppm), Co (7 ppm), Pb (13.33 ppm) have lower average concentrations than their crustal baseline average concentrations. Chemical index of alteration values for the samples also pointed to felsic source rock. The index of geoaccumulation values computed show that the following elements, namely: Sb, In, Zr are of serious environmental geochemical concern in the study area. However, heavy metals such as Cu, Cr, Co and Pb are below the pollution baseline. It is recommended that necessary environmental remediation methods should be deployed to minimise the impact of Sb, In and Zr pollution in the study area.

Keywords: Ogbagha sediments, environmental geochemistry, Okpella, enrichment ratios, geoaccumulation index

Biography

Dr. Solomon O. Obaje is a Nigerian born on 6th January, 1964 in Kaduna, Nigeria. He had his primary and secondary school education in Kaduna and Daura, Nigeria, respectively. He obtained B.Sc. (Hons.) degree in Geology in 1988 and M.Sc. Geology (Mineral Exploration/Geochemistry) in 2002, both degrees from the Ahmadu Bello University, Zaria, Nigeria. Also, he obtained Post Graduate Diploma in Computer Science in 2002 and Ph.D Geology in 2012 from the Federal University of Technology, Minna, Nigeria. He is the immediate past acting Head of Department of Earth Sciences in his university. He is married with two children.