A COMPARATIVE ANALYSIS OF DISCHARGE MEASUREMENT TECHNIQUES IN MAIGAMO TRIBUTARY OF RIVER KUBANNI, ZARIA, NIGERIA

BY

Mukhtar Suleiman1*, Yusuf Yakubu Obadaki2 and Abdullahi Jibril2

1Department of Geography

Faculty of Arts, Management and Social Sciences, Federal University Gashua, Yobe.

2Department of Geography and Environmental Management,

Faculty of Physical Sciences, Ahmadu Bello University, Zaria

*smukhtar27.sm@gmail.com; 07034994525

# ABSTRACT

River or stream discharge is of great importance ranging from flood control, power generation, irrigation, water supply and dam construction. Hence, the need for stream discharge measurement becomes a paramount importance in water resources evaluation.  A comparison has been made between stream discharge values using the Weir, Current metre and Float methods in Maigamo tributary which is found in the southernmost part of River Kubanni in Zaria.  This was made in order to establish the most effective method for measuring stream discharge for small rivers. The result observed from this study shows that the F-calculated (0.911) is less than the table value (3.09) at 0.05 confidence level. This means that there was no significant difference between the river discharges values obtained from the three techniques. This means that the techniques were suitable for the stream discharge measurements on the stream channel. However, the field observation shows that the weir technique is of greater applicability and more advantageous for continuous discharge measurements than the current metre and float methods. The weir technique can be used throughout the year and when the river becomes too shallow for current metre to be used. Also, the weir is of greater advantage compared to the float method, because the float is sometimes affected by vegetal growth, eddy current and wind. It is therefore recommended that for small river channels, the weir method is the most effective technique for stream discharge measurement.

Biography

Mukhtar Suleiman has completed his Msc at the age of 28 at Ahmadu Bello University Zaria, Nigeria. He specializes in Fluvial Geomorphology and Hydrology.He has published more than five papers at reputed journals and has been serving as an academic staff at Federal University Gashua Yobe, Nigeria.

Presenting Author Details

Full Name: Mukhtar Suleiman

Contact Number: +2347034994525

Session Name /Number:

Category: Oral presentation