MODELING MAGNETIC BASEMENT IN RELATIONSHIP TO HYDROCARBON HABITATS IN THE CENTRAL NIGER DELTA, NIGERIA.

By

OBI, D. A.1 ILOZOBHIE, A. J2. AND LEBO, S. E 3.

1&3. GEOLOGY DEPARTMENT UNIVERSITY OF CALABAR, NIGERIA 2.PHYSICS DEPARTMENT UNIVERSITY OF CALABAR. NIGERIACORRESPONDING AUTHOR Email: dominic_odu@yahoo.com

ABSTRACT

Aeromagnetics has over the years been used as a reconnaissance tool for hydrocarbon exploration. Modeling of source to magnetic anomalies can also reveal basin depth and configuration of supposed hydrocarbon habitats. This study was done using eight (8) aeromagnetic maps on a scale of 1:100,000 covering the central Niger Delta States in Nigeria. The Data was digitized and processed using the  United States Geological services potential  field software’s version 2.2 for map merging, reduction to pole, polynomial filtering, horizontal gradient magnitude (HGM), wenner doconvolution, forward and inverse modeling (pdep and saki programs). The result indicates areas around Benin River, Warri Kwale and Aboh with more dominantShallow depths (0.8km-2.01m) with minor areas reaching deeper sources (2.5-4.5km) while areas around Forcados, Burutu and Ahoda have predominantly deeper sources (2.5-4.5km). Generally there is an agreement in all the depth derived sources of an increase in sediment thickness accumulation from the onshore (Warri areas) towards the offshore (Burutu) with horst and graben structures as indicated from the saki models, the identified grabens are the recognized possible hydrocarbon habitat which forms potential targets for hydrocarbon exploration prior to seismic exploration.