SEQUENCE STRATIGRAPHY AND RESERVOIR STUDIES OF A COASTAL SWAMP DEPOBELT, NIGER DELTA, NIGERIA

Ijomah- Azubuike Kelechi

Department of Geology, University of Port Harcourt, Port Harcourt, Nigeria

ABSTRACT

Sequence stratigraphic concepts in defining sediments accumulation and preservation trends within the basin-fills have become a successful exploration technique in the search for hydrocarbons. This concept has been employed in coastal swamp depobelt in the Niger Delta. The study area has two recognized depositional sequences, bounded above and below by maximum flooding surfaces (MFS). The maximum flooding surfaces are Uvigerina 8, (9.5ma) and Dodo shale (11 .5ma) and the two sequence boundaries are l0.35ma and l0.6ma. Nonion 4, MFS (10.4ma) was missing in the section, which could have been as a result of 10.6ma erosion as evidenced from the seismic section. Based on the above, systems tracts were interpreted; High stand systems tract (HST), Low stand system tract (LST) and Transgressive systems tract (TST). The hydrocarbon bearing sand units were analyzed based on systems tracts. The LST had more hydrocarbon bearing sand units, followed by HST, while none was observed for the TST. The average porosity and permeability values for the systems tracts are; for the first HST, 0 = 38%, K = l2O2mD, Second HST, 0 = 36%, K = l52mD, LST 0 = 35%, K = 37n0 and the last HST 0 = 33%, K = 34mD. The reservoirs are more of gas bearing than oil. This is attributed to the source rock type, temperature effect and low permeability values (reduced pore throat).

BIOGRAPHY

Ijeomah-kelechi  is currently a doctoral research   student  at  Department of Geology, University of Port Harcourt. She has 10 Research publications in a Reputable research international journal.

PRESENTING AUTHOR DETAILS

Full name:  Ijomah- Azubuike Kelechi

Contact number: 234-8063936998

#### Session name/ number:  Recent upsurge in Geosciences and Remote Sensing: International Prospective

#### Category: (Oral presentation)

#### 2nd International Convention on Geosciences and Remote Sensing November 8-9, 2017 Las Vegas, Nevada, USA