ABSTRACT

INVESTIGATION OF TAR SANDS DEPOSITS IN IJEBU EAST OF OGUN STATE NIGERIA USING A SEISMIC REFRACTION METHOD

The interest in tar sands is great economic importances by the interest group have encouraged researches in the development of these resources. Vast tar sands resources have not been adequately explored and exploited by the Nigeria government. This motivated this study to map tar sand’s deposited in the studies area.

A seismic refraction survey at Imeri Ijebu Imushin Ijebu East was carried out using ABEM seismograph Terraloc Mark 6 version 2.22. Thirteen (13) traverses were taken along five (5) profiles in which both forward and backward shooting were measured to cover a spread of 72m in each of the traverse.

Pickwin software was adopted to analyze the results obtained in order to give information on first arrival time and velocities of the first and second layers. A model of the earth structure was achieved by the use of plotorrafa (refraction) software and interpretation to determine the thickness of the first layer from the processed data based on the known geology of the area. The velocities of the first layers of the five profiles ranges from (0.75 – 1.30)km/s and the second layers velocities ranges were (1.31 – 2.51)km/s.

The presence of the tar sands were observed in profile 1 (traverses 1 and 2); profile 3 (traverse 1) and profile 4. With a velocities value (1.12 – 1.44)km/s, (1.10 – 1.47)km/s and (1.12 – 1.86)km/s respectively. The depth of the first layers of the above profiles ranges from (5.00 – 7.10)m

BY

RAIMI ADEPOJU MUFTAU

E-MAIL ADDRESS: raimiadepojumufutau@gmail.com

SLT DEPARTMENT

FACULTY OF SCIENCE

OSUN STATE POLYTECHNIC, IREE

OSUN STATE, NIGERIA.