Paper Title

Poisson’s Ratio of Surface Soils and Shallow Sediments Determined from Seismic  Compressional and Shear Wave Velocities

Authours

Akaninyene  Okon Akankpo1 and Ubong Effiong Essien2

1Department of Physics, University of Uyo, Uyo, Nigeria.

2Department of Science Technology, Akwa Ibom State Polytechnic, Ikot Osurua, Nigeria.

Corresponding Authour

E-mails: akankpo@yahoo.com, akaninyeneakankpo@uniuyo.edu.ng

Phone Numbers: +2348023779058; +2348067584544

Abstract

This paper presents the results of seismic compressional, P- and shear, S-wave measurements carried out on the unconsolidated top-soil at the different locations in Akwa Ibom, Nigeria to determine Poisson’s ratio. Seismic refraction data was used to determine Poisson’s ratio as an aid to engineering foundation. A 12-channel seismograph with signal stacking ability was used together with high frequency (100 Hz) geophones on the top-soil. The geophone intervals were set to 5 m at the all locations. In all the locations, Vp/Vs ratio ranged from 1.0289 to 1.4185 for the top layer. Vp/Vs ratio in the second layer ranged from 1.0512 to 1.5834. The Poisson’s ratio for the first layer ranged from −8.0324 to 0.2060. For the second layer, the Poisson’s ratio ranged from −0.7567 to 0.1683. The values of Vp/Vs ratio less than 2 in the first layer and in some locations in the second layer resulted in negative Poisson’s ratio. The low and negative values of Poisson’s ratio are symptomatic of occurrence of ripable anisotropic materials in the locations where they occur, which suggests that indicated average depth should be removed and refilled with geomaterials that may be resilient to carry engineering loads.

Keywords

Seismic Refraction, Nigeria, Vp/Vs Ratio, Poisson’s Ratio

Biography of Presenter

The author holds Ph.D Degree in Geophysics from Michael Okpara University of Agriculture, Umudike, Nigeria. He is currently lecturing at the Department of Physics, University of Uyo, Nigeria. He has published not less than thirty articles in reputable journals both locally and internationally. He is a member of some professional bodies including Nigerian Institute of Physics (NIP) and Physics Writers Creation Series (PWCS).