NAME: FAITH NDUNGE SILA	INSTITUTION: KENYATTA UNIVERSITY

EMAIL: faithsila41@yahoo.com

THEME: SCIENCE TECHNOLOGY AND INNOVATION.

# Gravity And Seismic Structural Modelling Of Petroleum Systems Of

# Kerio Valley Basin, Kenya

ABSTRACT

Keywords: Vitrinite reflectance, modelling, Kerio Basin, Bouguer.

Kerio valley basin is one of the sedimentary basins in Kenya known to have hydrocarbon formation potential. Exploration is ongoing in the area with different geophysical methods being employed. The existence of such hydrocarbons developed interest in petroleum system structures and basin modelling in this research. The research utilized 2D seismic data and gravity data from National oil Corporation of Kenya with the aim of determining density variation within the basin, their characteristics and model petroleum system structures of Kerio Valley basin. Gravity data collected in the basin was processed and interpreted using Golden surfer 8 to provide information about rock density contrast forming the sedimentary layers resulting from deposition and compaction of rocks due to pressure as the basin evolved. Data was uploaded in Euler deconvolution software with structural index of 1 and 0.5 in window size. It was again loaded in GRAV2DC for forward modelling and the average density contrast values used were 2.2g/cm3,2.5g/cm3,2.28g/cm3 and 2.6g/cm3 respectively from the top to the most bottom stratigraphic layer formed due to deposition and compaction of different rocks over a period of time. Seismic data was interpreted in Petrel software, the value used for heat flow was 68Mw/m2 and paleo water depth were and 68m, 112m, 700m, 840m and 900m as model input. During this period, the basin passed through tectonic regime and transformed into a sedimentary basin. I-D model showed that the source rocks are mature and has potential to produce hydrocarbon as shown by vitrinite reflectance. Temperature curves shows that temperature was maximum during Miocene volcanics and has been decreasing over a period of time. Analysis showed that Kerio valley basin is a hydrocarbon potential area since it has a mature developed petroleum system.