Title: Pesticides in rural and urban atmosphere: concentrations and risk assessment.

Authors: Aleinnys M. Barredo Yera1, Madson M. Nascimento2 Gisele O. da Rocha2, Jailson B. de Andrade2 and Pérola de Castro Vasconcellos1.

1Institute of Chemistry, University of São Paulo. 2Institute of Chemistry, Federal University of Bahia.

Abstract

Since the 50s, the pesticides use has become essential for the development of the agriculture. These pesticides remain in the atmosphere for a certain time and depending of the compound, negative impact on the environment and in the human health can be observed. In Brazil, 24% of the gross domestic product (GDP) correspond to the agriculture and the state of São Paulo, particularly, is the second major consumer of pesticide of the country. In this work the concentration of pesticides in the atmosphere of São Paulo and Piracicaba (sugarcane plantation site) cities was determined and the assessment of the risk caused by inhalation of these compounds was calculated.

Samples (n=23) were extracted and 34 pesticides were determined by gas chromatography coupled to a mass spectrometer. The compounds with the highest detection frequency were permethrin I and II (96%). The concentrations ranged from 17 pg m-3 for tebuconazole in Piracicaba, to 166 pg m-3 for endrin aldehyde in São Paulo. The highest values of daily inhalation exposure were for heptachlor in infants, 8.29x 10-5 mg kg-1day-1 in Piracicaba and 5.00x10-5 mg kg-1day-1 in São Paulo. All values of the hazard quotients are lower than 1.0 indicating that there is no danger to human health by inhalation. Cancer risk was found for heptachlor in adults, infants and children in both sites.

References

- Scheyer. A, Morville. S, Mirabel. P, Millet. M. (2007). Variability of atmospheric pesticide concentrations between urban and rural areas during intensive pesticide application. Atmospheric Environment 41, 3604–3618.

- Casida. J and Durkin. K. (2013). Neuroactive insecticides: targets, selectivity, resistance, and secondary effects. Annual Review of Entomology 58, 99–117.

- MAPA. Ministério da Agricultura, Pecuária e Abastecimento. (2017). Agropecuária puxa o PIB de 2017. http://www.agricultura.gov.br/noticias/agropecuaria-puxa-o-pib-de-2017. Publicado 04/12/2017 18h24. Accessed: August, 16th 2018.

- Bombardi. L. (2017). Geografia do uso de agrotóxicos no Brasil e conexões com a União Europeia. FFLCH – USP. São Paulo