Design of a Water quality monitoring network for the Akaki Rivers in Addis Ababa, Ethiopia

Abstract

An important means of characterizing the health of streams is through measurement and assessment of the different chemical, physical and biological parameters. Thus cost effective and targeted water quality monitoring programs are required for proper assessment, restoration and protection of them. This research reports on the development of water quality monitoring for the Akaki Rivers in Addis Ababa, Ethiopia, in the eastern part of Africa. In this catchment, anthropogenic influences as well as natural processes are responsible for the degradation of surface waters and impairing of their use for various purposes. For the determination of the quality of water, various physico-chemical, biological and microbiological characteristics of 25 sites were studied in November, 2007 and January, 2008. Ecological parameters like dissolved oxygen (DO), pH, Nitrate (NO3-N), phosphate (PO4 3--P), Ammonia (NH3-N), biochemical oxygen demand (BOD5 20), chemical oxygen demand (COD), heavy metals, faecal coliforms and macro invertebrates were analyzed in-situ or in the laboratory and were compared with standard limits to investigate the influence of different sources of pollution for the final condition of the water. For that, the WHO, USEPA and STN 75 7221 guidelines for surface water quality were used. The obtained results indicate that almost all sites have low DO concentration, high number of faecal coliforms and elevated organic loads derived from discharge of untreated waste water, whichis responsible for the deterioration of water quality, heavy metals and low benthic macroinvertebrate taxa. The concentration of parameters like COD and BOD5 showed an increase in January compared to November, indicating that there was no dilution from rain water during this period. It was also found that it is possible to apply biological investigations using benthic macroinvertebrates for assessment of water quality condition,but additional investigation is required to adapt the index scoring to tropical ecosystem since it was developed for temperate environments. Aiming at a better understanding of water quality trends and a better water quality management, a systematic water quality monitoring network composed of 18 stations was proposed. Ambient, operational, effluent and early warnings are the main monitoring types recommended.

Key words: Akaki Rivers, Water quality monitoring, Water management.

Biography

I, am an Environmentalist, My     professional background is Chemistry in BSc and  MSc in Environmental Science   I  have  done  research on Water quality monitoring network of  the Akaki River and I was leading a research team at Addis Ababa Environmental Protection Authority. This team has done different researches like Industrial waste pollution level of the city of Addis Ababa by taking samples from the effluent of different Industries, Study of Vehicular air emission and also the pollutant discharged from Garages and hospitals to the ambient environment. I was leading on the guideline preparation on hazardous waste management of the city Addis Ababa Ethiopia. I have participated on different studies like the feasibility study of the common effluent treatment plant of the leather Industries

REFERENCES

Adriaanse, M. R. M. A. B. G. H. B. a. C. V. d. G. (2000).The Context of Assessment. In: Proceedings - International Workshop on Information for sustainable Water Management by J.G.Timmerman, W. cofino, R. Enderlein, W. Julich, P. Ross, N.Thyssen, R. Turner and R. Ward(eds), 425-427. Monitoring Tailor-Made III. Nunspeet.

Allan, I. J., B. Vrana, R. Greenwood, G. A. Mills, B. Roig and C. Gonzalez (2005).A " toolbox" forbiological and chemical monitoring requirements for the European Union's Water FrameworkDirective. Elsevier- Talanta 69: 302-322.

Allan, I. J. G. A. M. B. V. J. K. A. H. N. G. S. L. A. F. a. R. G. (2006).Strategic monitoring for theEuropean Water Framework Directive. Elsevier - Trends in Analytical Chemistry 25, N0 7.Florida.

ANZECC (2000).Australian and New Zealand Environmentat and Conservation Council, Australian and New Zealand Guidelines for Fresh and Marine Water Quality. [Web]. http://www.dhe.gov.au/water/quality/nwqms/summary/chapter2.html [Date of access: 19September 2006].

APHA/AWWA/WPCF (1985).Standard Methods for the Examination of Waters and Wastewaters 16thed. Washington, DC, USA.Armitage, P. D. D. M. J. F. W. a. M. T. F. (1983).The performance of a new biological water qualityscore system based on macroinvertebrates over a wide range of unpolluted running- water sites.Water research 17:333-347.

Azrina, M. Z. C. K. Y. A. R. I. A. I. a. S. G. T. (2005).Anthropogenic impacts on the distribution and biodiversity of benthic macroinvertebrates and water quality of the Langat River, Peninsular Malaysia. Elsevier - Ecotoxicology and Environmental Safety 64:337-347.

Bartram, J. a. R. B. (1996).Water Quality Monitoring - A practical guide to design and implementation of freshwater quality studies and monitoring programmes UNEP and WHO. London.

Bellos, D. a. T. S. (2005). Chemical pollution monitoring of the River Pinios (Thessalia - Greece). Elsevier - Journal of Environmental Management 76:282-292.

Benoit, R. J. (1971). Self - purification in natural waters. In: Water and Water Pollution Handbook by L.L.Ciaccio, vol.1.Dekker - New York.

I am now a director of Environmental Technology Directorate at Ethiopian Leather Industry Development Institute. This directorate is doing on Environmental management of Tanneries in the country whether they meet the require Environmental Standard or not. For this purpose the directorate gives consultancy service, and training for the industries. In addition there are different researches which are very crucial for the solid waste management of the Tanneries. This includes separation of fat and protein from the fleshing waste..