Title: Recycling of industry waste water for production of nanoparticles and its applications for waste water purification

Souad A. Elfeky

National Institute of Laser Enhanced Sciences, Cairo University, Giza, 12613, Egypt.

Waste acid resulted from the pickling process in steel industries have high concentration from iron ions that used in this research for synthesis of magnetic nanoparticles (Fe3O4) by co-precipitation method. Surface of these magnetic nanoparticles is modified by cyclodextrin (CD) polymer through the copolymerization reactions. Characterization of the synthesized CD /Fe3O4 nanocomposite was done by using Fourier Transform Infrared (FTIR) spectrometer, X-Ray Diffraction (XRD), and Transmission Electron Microscope (TEM). The generated polymer modified nanoparticles (CD /Fe3O4) was used for removing Pb(II), Ni(II), Cd(II), and Cr(VI) from wastewater samples.  Adsorption capacity of CD /Fe3O4 toward heavy metals was in the sequence of Cr(VI), Cd(II), Ni(II), and Pb(II) respectively in a single heavy metal system. In a binary system, Cr(VI) or Ni(II) removal efficiency was increased when each one paired with Pb(II) while Pb(II) removal increased when it paired with Cr(VI). Desorption reactions proved that this polymer can be reused many times with accepted efficiency.

Biography

Professor Elfeky has completed her PhD at the age of 32 years from Cairo University and postdoctoral studies from Bath and Birmingham Universities School of Chemistry. She was also the Dean and chairman of the high institute of Optics Technology, ministry of higher education, Egypt. She has published more than 30 papers in reputed journals and has been serving as an editorial board member of 3 reputed journals.

Presenting author details 
Full name: Souad Ahmed Farghly Zakaria Elfeky
Contact number: +2 01011007941

Link: https://scholar.google.com.eg/citations?user=xU5BwK8AAAAJ&hl=ar

Session name/ Cleaner production and Green Nanotechnologies number:02
Category: (Oral presentation)