HARVESTING ELECTRICAL ENERGY FROM ALOE VERA AND BANANA PLANTS

Nwogu C. N.

Department of Mechanical Engineering, Michael Okpara University of Agriculture Umudike, Nigeria. Email: checknolly@yahoo.com

ABSTRACT

Electrical energy was harvested from Aloe Vera (Aloe Barbadensis) and Banana (Musa Acuminate) plants. This was done by inserting pairs of electrodes (copper and zinc) into the plants to allow flow of ions thus, producing voltage at the terminals of the electrodes. Results of the experiment performed showed that Aloe Vera produced higher voltage 0.83v than Banana which produced a voltage of 0.74v. Also, it was seen that inserting a second and third electrode pairs in the same leaf branch produced voltage of 1.37v and 1.59v respectively for the Aloe Vera plant and 0.86v and 1.11v respectively for the Banana plant while inserting a second and third electrode pairs in separate leaf branches produced output voltages of 1.4v and 2.01v for the Aloe Vera plant and 0.92v and 1.15v for the Banana plant respectively. Finally, it was proven experimentally that sunlight had very little effect on the output voltage.

Keywords: Aloe Vera, Banana, Electrical energy, Electrode, Voltage.