# Treatment of produced water in oil and petroleum extraction

Grazia Leonzio

Department of Industrial and Information Engineering and Economics, University of L'Aquila, Via Giovanni Gronchi 18, 67100 L'Aquila, Italy;

Oil content and salinity of produced water from offshore and onshore activities during the extraction of gas and oil can be reduced through various physical, chemical, and biological methods [1]. Infect, produced water is the largest waste stream generated in oil and gas industries and it is a mixture of different organic and inorganic compounds.

In this research, an evaporation plant with mechanical vapor recompression (MVR), in figure 1, is compared with an evaporation plant with boiler, in figure 2, in order to desalinate the produced water. The plant has an evaporation capacity equal to 0.025 kg/s. A simulation of the process in ChemCad is carried out. The feed is 1 m3/h flow rate and has the following composition: water 96.6 %w/w, NaCl 3.3 %w/w, benzene 9.9∙10-3 %w/w, toluene 1.7∙10-2 %w/w.

Results show that the feed temperature does not influence the process, while the vacuum degree and the vaporization fraction play a crucial role on energetic and investment expenditure and on the disposal costs, due to the rich solution of NaCl and pollutants. A sensitivity analysis determines the optimal operating conditions: temperature of feed of 298 K, vacuum degree 0.37 bar, vaporization fraction 46.5%, compressor power 34.7 kWh, evaporator area 7 m2 and concentration factor of 1.83. In this conditions, the disposal cost is equal to 160 €/h, the energetic cost is 8.8 €/h while the evaporator costs is 100000 €. Comparing the same plant with a boiler, results show that the process provides a lower specific energy expenditure equal to 10.7 €/m3 while the plant integrated by a boiler provides 16.7 €/m3 of energy expenditure. Future researches should verify the obtained results in a real plant.

Figure 1 Scheme of the desalination plant integrated by MVR system in ChemCad environmental

Figure 2 Scheme of the desalination plant integrated by a boiler system in ChemCad environmental

References

- Ahmaduna FR, Pendashteha A, Abdullaha LC, Biaka DRA, Madaenic SS, Abidina ZZ (2009) Review of technologies for oil and gas produced water treatment, Journal of Hazardous Materials 170:530–551.

Biography

Grazia Leonzio is a PhD student from L’Aquila University. She published several articles and participated to several international and national congresses about environmental and energy aspect of chemical processes. She wrote an article about waste management in Italian regions and published in Columbia University web-side. She participated to M.U.N conferences and she is a member of several associations: A.I.D.I.C. (Italian Association of Chemical Engineering), S.C.I. (Italian Chemical Society), I.S.S.N.A.F. (Italian Scientists and Scholars in North America), E.C.A.S. (European Commission Authentication Service). She is a referee of several journals.

Email: grazia.leonzio@graduate.univaq.it

Notes/Comments

Presenting author details

Full name:  Grazia Leonzio

Contact number: 3401537804

Mail id: grazia.leonzio@graduate.univaq.it

Category:  Poster presentation