Experimental and computational studies on zeolite-Y encapsulated iron(III) and nickel(II) complexes containing mixed-ligands of 2,2- bipyridine and 1,10-phenanthroline

Solomon Legese Hailu, Tshwane University of Technology, Department of Chemistry, South Africa

### Abstract (300 word limit)

Figure 1: Flexible Ligand synthesis of mixed ligand

complex of 2,2'-bipyridine and 1,10-phenanthroline

with iron (III) and nickel (II) with its catalytic

application towards oxidation of 2-phenyl phenol

Statement of the Problem: The growing demands for the design of environmentally friendly products and processes have led to the development of environmentally friendly catalytic system with green chemistry concepts. Furthermore, in view of the growing concern over environmental pollution and with further advance of catalytic sciences, chemical industries are forced to adopt new safe and eco-friendly wastewater treatment technologies with less consumption of energy for their waste treatment process. The purpose of this study is to explain the use of zeolite based heterogeneous catalysis for the efficient and environmentally friendly treatment of industrial chemical pollutants. Methodology & Theoretical Orientation: Mixed ligand complex of 2,2'-bipyridine and 1,10-phenanthroline with iron (III) and nickel (II) have been encapsulated into zeolite cage by the reaction of zeolite exchanged metal ion with flexible ligands. Density functional theory calculation has being carried out on both neat complexes as well as metal complexes encapsulated into NaY zeolite to investigate change in structural parameters, energies of the HOMO and LUMO, global hardness and softness of the two metal complexes upon encapsulation into zeolite. Findings: Experimental results confirm successful formation of mixed ligand complex of Fe(III) and Ni(II) inside the zeolite supercage. Density functional theory (DFT) calculations predict higher reactivity of the zeolite encapsulated metal complexes compared to respective metal complexes. The zeolite encapsulated Fe(III) and Ni(II) complexes are found to be catalytically active for the decomposition of hydrogen peroxide and for the oxidation of 2-phenyl phenol with good efficiency. The reusability test on the catalyst also confirms that zeolite encapsulated metal complex could be used more than three times without losing their catalytic efficiency Conclusion & Significance: The zeolite encapsulated metal complexes could be used as a green catalyst for treatment of industrial organic pollutants.

### Image

Recent Publications (minimum 5)

- Solomon L. Hailu, B. Nair, M. Redi-Abshiro, R. Aravindhan, I. Diaz, M. Tessema, (2015) Synthesis, characterization and catalytic application of zeolite based heterogeneous catalyst of iron(III), nickel(II) and copper(II) salen complexes for oxidation of organic pollutants. Journal of Porous Materials, 22 : 1363-1373

- Solomon L. Hailu, B.U. Nair, M. Redi-Abshiro, R. Aravindhan, I. Diaz, M. Tessema, (2015) Experimental and computational studies on zeolite-Y encapsulated iron(III) and nickel(II) complexes containing mixed-ligands of 2,2′-bipyridine and 1,10- phenanthroline. RSC Advances, 5 : 88636-88645

- Solomon L. Hailu, B.U. Nair, M. Redi-Abshiro, R. Aravindhan, I. Diaz, M. Tessema (2016) Oxidation of 4-chloro-3-methyl-phenol (PCMC) through zeolite Y encapsulated Catalyst of Iron(III), Nickel(II) and Copper(II) N,N′-disalicylidene-1,2- phenylenediamine Complex. Chinese Journal of Catalysis, 37 : 135-145

Biography

Dr. Solomon Legese Hailu earned his BSc in Chemistry, MSc in Environmental Science and PhD in Chemistry form Addis Ababa University. Dr. Solomon has an area of expertise in synthesis and characterization of environmental friendly green catalytic systems for application of industrial wastewater. His new developed zeolite based mixed ligand of metal complex catalytic system were applied for tanning industries for treatment of organic pollutants.

Email: birllpti.2001@gmail.com