Simultaneous removal of heavy metals and ciprofloxacin micropollutants from wastewater using EDTA functionalized b-cyclodextrin-chitosan adsorbent

Monu Verma*, Hyunook Kim*

Water-Energy Nexus Laboratory, Department of Environmental Engineering, University of Seoul, Seoul, 02504, Republic of Korea

### Abstract: The emerging inorganic and organic micropollutants in water system have raised great concern due to their harmful impact on human health and environmental system. The present study pertains the synthesis of EDTA functionalized cyclodextrin-chitosan (CDCSEDTA) composite via two-step process and employed in the adsorptive removal of toxic inorganic (Pb(II), Cu(II) and Ni(II)) and antibiotic micropollutant ciprofloxacin (CIP) from aqueous solution. Characterizations of the CDCSEDTA were carried out using FESEM, EDX, XRD, FTIR, and TGA to identified different characteristics such as surface morphology, elemental composition, functionalities, amorphous nature, and thermal stability, respectively. Thereafter, different batch adsorption experiments were applied to attain the adsorption properties. Kinetics results fit well with pseudo-second order (PSO) kinetics model for both inorganic and organic micropollutants. However, adsorption of heavy metal ions to the adsorbent was faster than that of CIP. Isotherms results showed excellent monolayer adsorption capacities of 330.90, 161, and 118.90 mg g-1 for Pb(II), Cu(II), and Ni(II), respectively, while heterogeneous adsorption capacity of 25.40 mg g-1 for CIP.. Adsorption mechanism was investigated using EDX, elemental mapping and FTIR which confirmed the chelation with EDTA and electrostatic interaction of OH and NH2 groups with metal ions, while CIP was adsorbed via electrostatic attraction and inclusion complex formation with CD cavities of adsorbent. More significantly, adsorbent gave good removal efficiencies during simultaneous adsorption of metal ions and CIP at environmental level concentrations from real wastewater. Furthermore, excellent reusability makes it viable alternative to remove the inorganic and organic micropollutants for wastewater treatment.

### Keywords: Heavy metals, Kinetics, Isotherms, Micropollutants, Simultaneous adsorption, Wastewater treatment.
Image

Recent Publications (minimum 5)

1. Monu. Verma, Multifunctional β-cyclodextrin-EDTA-chitosan polymer adsorbent synthesis for simultaneous removal of heavy metals and organic dyes from wastewater. Environmental Pollution 292 (2022) 118447.

2. Monu Verma, Synthesis of EDTA-functionalized graphene oxide-chitosan nanocomposite for simultaneous removal of inorganic and organic pollutants from complex wastewater, Chemosphere 287 (2022) 132385.

3. Monu Verma, Fabrication of GO–MnO2 nanocomposite using hydrothermal process for cationic and anionic dyes adsorption: Kinetics, isotherm, and reusability, Journal of Environmental Chemical Engineering 9 (2021) 106045.

4. Monu Verma, Graphene oxide-manganese ferrite (GO-MnFe2O4) nanocomposite: One-pot hydrothermal synthesis and its use for adsorptive removal of Pb2+ ions from aqueous medium, J. Mol. Liq. 315 (2020) 113769.

5. Monu Verma, Hyunook Kim, Graphitic bio-char and bio-oil synthesis via hydrothermal carbonization-co-liquefaction of microalgae biomass (oiled/de-oiled) and multiple heavy metals remediations, Journal of Hazardous Materials 409 (2021) 124987

Monu Verma has expertise in the field of environmental engineering issues, specially related to wastewater. His open and contextual evaluation model based on materials synthesis for the column preparation to purify the water through adsorption process. Apart from it, he is working in biomaterials uses to solve the environmental related issues. He published more than 22 articles in Sci and Scopus journals related to solve the environmental problems. Currently, he is working as a Brain Pool Fellow in University of Seoul, South Korea.

Email: mkvcyiitr@gmail.com