High Sensitive GC-MS Determination of Polycyclic Aromatic Hydrocarbons in Water Coupled  with Electroflotation-assisted Emulsification Liquid–Liquid Microextraction

Krylov Valentin A.

N. I. Lobachevsky Nizhny Novgorod State University, 603950, Russia

Polycyclic aromatic hydrocarbons are priority pollutants due to the very high toxicity. Therefore, to determine them, it is necessary to use sensitive methods with preconcentration. In the present study, a novel method named electroflotation-assisted emulsification liquid–liquid microextraction combined with GC-MS analysis has been proposed for the preconcentration and determination of polycyclic aromatic hydrocarbons in water samples. The advantage of electroflotation demulsification is the ease of changing the gas flow and size of gas micro-bubbles. The formation of gas micro-bubbles occurs on platinum electrodes soldered into a glass concentrator. Hexane, toluene and o-xylene were used as extractants. Application of extract capillary collection have solved the problem of the light extractant sampling. Dispersion of the extractant was performed by ultrasound. The volume of microextracts was 7-10 µl. The recovery of polycyclic aromatic hydrocarbons from water was 62-95%. A DB-5 (5% phenyl + 95% polydimethylsiloxane) fused-silica capillary column (30 m ×0.25 mm i.d. and 0.25-µm film thickness)was applied for separation of the analytes. The limits of detection and quantification of polycyclic aromatic hydrocarbons achieved were at the level of 10-5–10-6 mg/L and  highly competitive with the best world results. The methods of accounting or elimination of systematic errors are proposed. Purification of solvents by Rayleigh distillation method allowed to obtain samples with impurity content lower than (1-4)∙  10-3 mg/L. Containers for sampling and storage of samples to be analyzed should be made of borosilicate glass or quartz. The expanded uncertainty of determination was calculated. It included precision, uncertainty of standards preparation, calibration, sample introduction, enrichment factor. The relative expanded uncertainty was at the level of 13-30%.

Biography. Krylov V. A. has completed his PhD from  Gorkiy State University, Russia. He is head of the division of analytical chemistry of the Nizhny Novgorod State University. The main direction of scientic research of professor Krylov is the development of the theory and applications of chromatography for the analysis of high purity substances, environmental objects and development of methods of the microextraction. He has over 200 scientic papers, including reviews on the analytical chemistry of air, high purity volatile substances and liquid-liquid microextraction. He presented papers in more than 50 russian and international conferences.

Presenting author details 
Full name: Krylov Valentin A.
Contact number:
Twitter account:
Linked In account:

Session name/ number: Mass Spectrometry in Food Analysis

Category: Oral presentation