Analysis of acetylated peptides: Implications in proteomics

M V Jagannadham*

*Chief Scientist, CSIR-Centre for Cellular and Molecular Biology, Hyderabad-500007, India

Abstract

Sequence determination of peptides using mass spectrometry plays a crucial role in the bottom-up approaches for the identification of proteins. It is important to minimise false detection and validate sequence of the peptides in order to correctly identify a protein. The MS/MS spectra obtained are often with incomplete fragmentation and poor spectral quality. Chemical modification of peptides followed by mass spectrometry is another method for improving the spectral quality.  In silico derived tryptic peptides with different N-terminal amino acids were designed from human proteins, synthesized and analysed using LC coupled ESI-MS/MS. The effect of acetylation on the fragmentation of peptides was also studied. N-terminal acetylation of the tryptic peptides was shown to form b1-ions, improve the abundance and occurrence of b-ions. In some cases, the intensity and occurrence of some y-ions also varied. Thus, acetylation was found to be a fragmentation directed chemical modification that improves the efficiency of sequence determination of peptides. It is demonstrated that acetylation plays an important role in improving the de novo sequencing efficiency of the peptides. Acetylation is a simple reaction that can be carried out on a mixture of peptides as is required in proteomics. The acetylation method was extended to tryptic peptides generated from the proteome of an Antarctic bacterium Pseudomonas syringae Lz4W using the proteomics work flow and  mass spectra of the peptides were analysed. Comparison of the MS/MS spectra of the acetylated and unacetylated peptides revealed that acetylation helped in improving the spectral quality and validated the peptide sequences. Using this method, 673 proteins of the 1070 proteins identified were validated. Acetylation is found to be a good peptide sequence validation method for a large number of identified proteins.