Abstract

Statement of problem:

Poly (methyl methacrylate) (PMMA) obturator bulbs are hard and stiff. They tend to abrade the delicate tissues in maxillary defects. Relieving these obturators limits the support available for the bulbs and may limit mastication on the defect side. Resilient liners may improve mastication by improving support from closer contact.

Purpose:

The purpose of the study was to compare masticatory function scores of individuals with maxillectomies rehabilitated with all PMMA and silicone resilient liner lined PMMA obturator bulbs.

Materials and methods:

Twelve participants with maxillectomies were provided with two definitive obturator dentures after initial rehabilitation. One of the  obturators  had a hollow obturator bulb in all PMMA, while the other had a core of hollowed out PMMA with an outer layer of silicone resilient liner.

The participants wore each obturator for a two week period without wash out. Masticatory function on the defect side was assessed with an adapted Sato questionnaire. Statistical analysis was with a paired t-test on mean masticatory function scores. The level of significance was set at p<.050.

Results:

The participants found 98.35% of foodstuffs easy to masticate on the dentate side. An average of 60.40% (13.90%) of patients found the foodstuffs easy to masticate on the defect side with the silicone lined obturators. 18.35% (19.10%) of patients found the foodstuffs easy to masticate with the all PMMA obturators. Data were significant at p< .050.

Conclusion:

Resilient lining of polymethyl metacrylate resin obturator bulbs significantly improves masticatory ability in rehabilitated areas of the mouth.