# Deep space radiation like hardness of VO2 based active smart radiation device for nano satellite applications

Itani G. Madiba, Unisa/iThemba LABS, South Africa.

Nicolas Emond, INRS, Canada

Artur Braun, Empa, Switzerland

Malik Maaza, Unisa/iThemba LABS. South Africa

### Abstract

Vanadium dioxide (VO2) is explored as an active smart radiation device  (SRD) for new type of thermal control material for space craft especially in nano- and micro satellites.  Current space thermal control systems require heaters with an additional power penalty to maintain space craft temperatures during cold swings. Because its emissivity can be changed without electrical instruments or mechanical part, the use of VO2 based SRD decreases the request of spacecraft power budget. As an active SRD, an ideal VO2 based nano-coatings should have a low emissivity at low temperatures to maintain the heat, whereas at high temperatures its emissivity should be high to dissipate the additional unnecessary heat. In addition to their active thermal management properties, these VO2 nano-coatings must exhibit deep space radiations hardness. while in outer space such coatings will be exposed to different cosmic radiation including gamma rays and Neutrons. Because these radiations interact with the material in different ways, this contribution reports on the study of VO2 coatings properties subjected to extensive gamma rays and neutrons with typical doses to space missions.

Recent Publications

- F.J. Morin, Oxides which shows the metal-Insulator phase transition at the neel temperature, Phys. Rev. Lett. 3 (1959) 34-36.

- M. Benkahoul, M. Chaker, J. Margot, E. Haddad, R. Kruzelecky, B. Wong, W. Jamroz, P. Poinas, Thermochromic VO2 film deposited on Al with tunable thermal emissivity for space applications  Sol. Energy Mater. Sol. Cells 95 (2011) 3504–3508

- E. Haddad, R. Kruzelecky, B. Wong, W. Jamroz, M. Soltani, M. Chaker, P. Poinas, M. Benkahoul, Multilayer Tuneable Emittance Coatings with Low Solar Absorptance for Improved Smart Thermal Control in Space Applications "Society of Automotive Engineers SAE, (2009) 2009-01-2575 SAE Inc. 1 (2009) 2575–2587.

- G.I. Petrov, V.V. Yakovlev, J.A. Squier, Non linear optical microscopy of ultrafast phase transformation in vanadium dioxides. Opt. Lett. 27 (2002) 655-657.

- M. Maaza, S. Simo, B.D. Itani, J.B. Kana Kana, E.L. Harti, K. Boziane, M.L. Saboungi, T.B. Doyle, L. Lukyanchuk, Phase transition in a single VO2 single crystal: potential femtosecond tunable opto-electronic nano-gating, J. Nanopart. Res 16 (2014) 2397-2401.

Biography (150 word limit)

Itani Given Madiba has his expertise on nanoscience and nanotechnology. Synthesis and development of nano-materials for energy and space applications.

Email: itanibravo@tlabs.ac.za

Notes/Comments: