# Enhancing the photocatalytic activity for pollutant degradation and H2 evolution by modifying ZrO2 with nanoclusters of BiVO3, Ag3PO4, SrTiO3 and WO3: A first-principles study

Francis Opoku1, Krishna Kuben Govender2, Cornelia Gertina Catharina Elizabeth van Sittert3, Penny Poomani Govender1*

1Department of Applied Chemistry, University of Johannesburg, P. O. Box 17011, Doornfontein Campus, 2028, Johannesburg, South Africa

2Center for High Performance Computing, 15 Lower Hope Road, Rosebank, Cape Town, 7700

3Research Focus Area for Chemical Resource Beneficiation: Laboratory of Applied Molecular Modelling, North-West University, Potchefstroom, 2520, South Africa

*Corresponding author. Tel: (+27) 011-5596555; Fax: (+27) 011 559 6425; E-mail: pennyg@uj.ac.za

### Abstract

Environmental pollution and energy exhaustion has received much interest both in scientific and industrial research fields. For this reason, tremendous efforts are made to design novel environmentally friendly non-polluting and sustainable energy storage photocatalysts for efficient solar energy harvesting and conversion. Semiconductor-based photocatalysis has received increasing attention in energy storage and environmental remediation process due to the abundantly solar energy. For this purpose, several heterostructures using BiVO4, Ag3PO4, SrTiO3 and WO3 monolayers coupled with ZrO2 nanocluster are designed to examine their potential applications in energy storage and degradation of pollutants using density functional theory calculations for the first time. Moreover, the underlying mechanism, band edge positions, optical and electronic properties of the ZrO2-based heterostructures are evaluated. The results displayed that the calculated band gap of the heterostructures is reduced compared to the pure ZrO2, which favour redshift absorption. A type-I band alignment was attained for the BiVO4/ZrO2, Ag3PO4/ZrO2 and WO3/ZrO2 heterostructures. More importantly, the type-II staggered band alignment formed in the SrTiO3/ZrO2 heterostructure restrained the charge recombination rate of the photoinduced carrier charges, as well as enhancing the photocatalytic activity. Our results display efficient charge separation and visible light response of the BiVO4/ZrO2, Ag3PO4/ZrO2, WO3/ZrO2 and SrTiO3/ZrO2 heterostructures. In particular, suitable band alignment of SrTiO3/ZrO2 with enough driving forces for charge carrier transfer show overall water splitting and degradation of pollutant in which SrTiO3 acted as the charge separation centre. Thus, the SrTiO3/ZrO2 heterostructure emerges as a new type of ZrO2-based photocatalyst for efficient solar energy applications. Furthermore, h+, HO• and O2−• radicals played a major role in the photocatalysis process. Finally, possible charge separation and photocatalytic mechanisms of BiVO4/ZrO2, Ag3PO4/ZrO2, WO3/ZrO2 and SrTiO3/ZrO2 heterostructures are proposed.

Figure 1. Schematic of the photodegradation mechanism of organic pollutants in water resources.

Recent Publications (minimum 5)

- Opoku F, Govender KK, van Sittert CGCE, Govender PP (2017) Recent progress in the development of semiconductor-based photocatalyst materials for applications in photocatalytic water splitting and degradation of pollutants. Advanced Sustainable Systems. In Press. DOI: 10.1002/adsu.201700006.

- Agorku ES, Kwaansa-Ansah EE, Voegborlo RB, Amegbletor P, Opoku F (2016) Mercury and hydroquinone content of skin toning creams and cosmetic soaps, and the potential risks to the health of Ghanaian women. SpringerPlus 5:319.

- Boateng TK, Opoku F, Acquaah SO, Akoto O (2016) Groundwater quality assessment using statistical approach and water quality index in Ejisu-Juaben Municipality, Ghana. Environmental Earth Sciences 75:1-14.

- Nkansah MA, Opoku F, Ackumey AA (2016) Risk assessment of mineral and heavy metal content of selected tea products from the Ghanaian market. Environ Monit Assess 188:1-11.

- Nkansah MA, Opoku F, Ephraim JH, Wemegah DD, Tetteh LP (2016) Characterization of Beauty Salon Wastewater from Kwame Nkrumah University of Science and Technology, Kumasi, Ghana, and Its Surrounding Communities. Environ Health Insights 10:147.

- Opoku F, Asare-Donkor NK, Adimado AA (2014) Theoretical study of the gas-phase decomposition of Pb[(C6H5)2PSSe]2 single-source precursor for the chemical vapour deposition of binary and ternary lead chalcogenides. Can J Chem 93:317-325.

- Opoku F, Asare-Donkor NK, Adimado AA (2015) Quantum mechanical study of the kinetics, mechanisms and thermodynamics of the gas-phase decomposition of Pb[(iPr)2PSSe]2 single-source precursor. J Organomet Chem 787:33-43.

- Opoku F, Asare-Donkor NK, Adimado AA (2015) Theoretical studies of the decomposition of Zn[(i Pr)2PSSe]2 single-source precursor in the gas phase for the chemical vapor deposition of binary and ternary zinc chalcogenides. Comput Theor Chem 1058:1-11.

- Opoku F, Asare-Donkor NK, Adimado AA (2015) Thermal decomposition of Zn[(C6H5)2PSSe]2 single-source precursor for the chemical vapour deposition of binary and ternary zinc chalcogenides: a theoretical study. SpringerPlus 4:266.

- Opoku F, Asare-Donkor NK, Adimado AA (2016) A comprehensive understanding of the chemical vapour deposition of cadmium chalcogenides using Cd[(C6H5)2PSSe]2 single-source precursor: a density functional theory approach. Chem Cent J 10:4.

- Opoku F, Asare-Donkor NK, Adimado AA (2014) Density functional theory (DFT) study of the gas-phase decomposition of the Cd[(iPr)2PSSe]2 single-source precursor for the CVD of binary and ternary cadmium chalcogenides. J Mol Model 20:2484.

Francis Opoku received his BSc. (Hons) Chemistry (2010) and M.Phil. Inorganic Chemistry (2014) degrees from the Kwame Nkrumah University of Science and Technology, Ghana. He is now pursuing a PhD degree in Chemistry under the supervision of Dr Penny Poomani Govender, Dr Krishna Kuben Govender and Dr Cornelia Gertina Catharina Elizabeth van Sittert in the Department of Applied Chemistry, University of Johannesburg, South Africa. His research interests include the design of efficient semiconductor-based photocatalyst materials and their applications in water splitting as well as degradation of pollutants in wastewater/water resources.

Email: ofrancis2010@gmail.com