Characterization and application of new ultrafiltration ceramic membranes for water treatment

A.Chougui, A. Belouatek

Laboratoire SEA2M, Université Abdelhamid ibn badis de Mostaganem, Algeria 27000

Abstract

We developed and tested twonew ultrafiltration ceramic membranes synthesizedfrom a locally available barbotine, kaolin, zirconia (ZM01 membrane) and 3-mercaptopropyltrimethyoxysilane (AZM01).The electrokinetic characterization of membranes highlighted a negative charge density over a wide range of pH. The addition of 3-mercaptopropyltrimethyoxysilane led to a decrease in water permeability from around 95 to 35 L.h-1.m-2.bar-1.

For both membranes salt and dye rejections were found to be strongly influenced by electrostatic interactions with the membrane surface, leading to a loss of performance with increasing the solution ionic strength. The maximum dye rejections were obtained with the AUF01 membrane and were close to 100 % and 80 % for cationic (methyl greenand neutral red) and anionic (reactive black5) dyes, respectively.

Keywords: ceramic membranes, dyes, zeta potential, electrostatic interactions, filtration.