# Effect of nano-zones on thermal shock behavior of mixed composite top coat APS TBCs

A. Ashofteh, PhD, University of Tehran, Iran

S. Seifollah pour, MSc, University of Tehran, Iran

### Abstract

Effects of nano-zones on the performance of thermal barrier coatings (TBCs) with the mixed composite top coat (MCC) were investigated under thermal shock conditions. To produce MCCs, firstly, the powders are mixed with the specified weight ratio, and then the prepared mixture is fed to the plasma stream of the atmospheric plasma spray (APS) machine. Ceria-yttria stabilized zirconia (CSZ) and micro- and nano-structured yttria stabilized zirconia (YSZ and YSZ-N) were used to produce coating samples. The coatings were grouped in Y-C and YN-C classes, and each class was produced with two different weight ratios and two different thicknesses. The results show that the presence of YSZ-N plays an important role in increasing the life of the samples and decreasing the thickness of the thermally grown oxide layer.

### Recent Publications

- Ashofteh, A., et al., Effect of layer thickness on thermal shock behavior in double-layer micro-and nano-structured ceramic top coat APS TBCs. Ceramics International, 2017.

- X. Song et al., "Thermal stability of yttria-stabilized zirconia (YSZ) and YSZ-Al2O3 coatings," Ceramics International, vol. 43, no. 16, pp. 14321-14325, 2017/11/01/ 2017.

- X. Fang, G. Zhang, and X. Feng, "Performance of TBCs system due to the different thicknesses of top ceramic layer," Ceramics International, vol. 41, no. 2, pp. 2840-2846, 2015.

- R. Ghasemi and H. Vakilifard, "Plasma-sprayed nanostructured YSZ thermal barrier coatings: Thermal insulation capability and adhesion strength," Ceramics International, vol. 43, no. 12, pp. 8556-8563, 2017.

- Y. Chai, C. Lin, and Y. Li, "Effects of Creep-Plastic Behavior on Stress Development in TBCs during Cooling," Ceramics International, 2017.

- F. Zhou et al., "Thermal cycling behavior of nanostructured 8YSZ, SZ/8YSZ and 8CSZ/8YSZ thermal barrier coatings fabricated by atmospheric plasma spraying," Ceramics International, vol. 43, no. 5, pp. 4102-4111, 2017.

Biography

Dr. Afshin Ashofteh has his expertise in evaluation and improvement of thermal barrier coating of turbine blades. His focus is on designing the structure of composite TBCs (mixed- and layer-composites) and to investigate the effect of the involved factors in these coatings on its performance. He has chosen this area after years of research and work on high temperature coatings, and he believes that composite thermal barrier coatings can overcome many of the existing limitations.

Email: a.ashofteh@ut.ac.ir