Title: Growth mechanism, structural and compositional properties of Cu2ZnSnS4 (CZTS) thin films deposited by RF sputtering method from a compound target.

S. Abdullahia, M. Momoha, A.U. Moreha, A. M. Bayawab and O. Popoolac

a) Department of Physics, Usmanu Danfodiyo University Sokoto, Nigeria

(b) Department of Pure and Applied chemistry, Usmanu Danfodiyo University Sokoto, Nigeria.

(c) Department of Mechanical Engineering, Florida International University, Miami, USA.

Kesterite-type Cu2ZnSnS4 (CZTS) thin films were deposited on corning glass from a single quaternary target. In this study, we investigated the growth mechanism and the influence of film thickness on the structural and compositional properties of CZTS films. All the four samples (as-deposited inclusive) show peaks corresponding to the kesterite-type structure. The diffraction peaks of (112) are sharp and the small characteristic peaks of the kesterite structure such as (220)/(204) and (312)/(116) are also clearly observed in X-Ray diffraction patterns. Besides, the films exhibit a high absorption coefficient of the order of 104 cm-1, an average optical band gap of 1.56 eV, an average resistivity of 290 ×104 Ω-cm. These results indicate that the quaternary CZTS would be a potential candidate for solar cell applications.

Biography

Sanusi Abdullahi, a PhD student has completed his Master’s degree from Usmanu Danfodiyo University Sokoto in 2012. He has a number of publications in the the area of thin film depositions and characterization.

Presenting author details

Full name: Sanusi Abdullahi

Contact number: +2347030394023

Category: Oral presentation

Keywords: RF sputtering; Cu2ZnSnS4 thin film; annealing; growth mechanism; renewable energy.