The Investigation of Polarization Characteristics of hyperbolic metamaterials

Le Cheng, Tao Wang*

Wuhan National Laboratory for Optoelectronics, Huazhong University of Science and Technology, Wuhan 430074, China

ABSTRACT

Polarization manipulation of the electromagnetic wave is essential for numerous and extensive applications such as biological imaging, sensing, communication and microscopy. We propose a polarization control system based on graphene-dielectric multilayer stacking arrays and investigate the characteristics of transmission spectrum and phase shift. For the proposed subwavelength graphene-dielectric multilayer structure, each stack can be homogenized and regarded as hyperbolic media via the effective medium theory. In our work, we have calculated spectral responses and phase change using the finite-difference time-domain (FDTD) method. The proposed structure exhibits ultrahigh confinement for electromagnetic wave, meanwhile, this homogenized hyperbolic material also shows that the position of the resonance frequency is extremely sensitive to the geometrical parameters of the proposed structure. The results show a high extinction ratio polarizer, and the operating frequency of which can be designed via changing the geometric parameters and Fermi level. Owing to the strong anisotropy of the proposed structure and the design of rectangular arrays, different polarization states have different transmission characteristics, resulting in a large phase difference between p-polarized and s-polarized light. Therefore, a large phase difference is generated in the x and y directions, and polarization control can be achieved by the superposition of the two modes along the orthogonal directions. It is found that the ellipticity is tunable by polarization angle demonstrating the excellent performance of polarization conversion. The proposed structure demonstrates excellent polarization manipulation capability, ultrathin thickness and flexible control of the operating frequency, providing a good avenue for integration of device in the infrared.

Biography

Tao Wang has completed his PhD at the age of 25 years from the key laboratory of laser technology, Huazhong University of Science and Technology, P. R. China. He is the Professor of Huazhong University of Science and Technology. He has published more than 55 papers in reputed journals and has been serving as an Member of SPIE from January 1997 and the Chinese Optical Society from 1990. His research interests include Optoelectronic Devices and Integration, Biological Optical Sensing, Information Optics and Photonics Technologies, Nonlinear Optics and Applications, Nanophotonics, Nanostructures and Nanometrology,

Presenting author details

Prefix ( Prof. )

First Name: TAO

Middle Name:

Last (Family) Name: WANG

Contact number: 86-13545003272

Email Address: wangtao@hust.edu.cn

Twitter account:  No

Linked In account: No

Session name/ number: Nano Science and Technology, Nanophotonics

Category: Poster presentation