Title: Effect of Ionizing Radiation on Carbon Nanotubes and its Application to the Development of Reversible Radiation Sensors.

1Kenneth Fontánez, 2Abniel Machín, 1Maria C. Cotto,

1José Duconge, 3Carmen Morant and 1Francisco Márquez

1Nanomaterials Research Group-NRG, Department of Chemistry and Physics, School of Natural Science and Technology, Universidad Ana G. Méndez-Gurabo Campus, 189 St.  Rd. Km 3.3, 00778-Gurabo, PR, USA

2Department of Chemistry, University of Puerto Rico, Rio Piedras Campus, 17 Ave. Universidad, 00925-San Juan, Pr, USA

3Department of Applied Physics, Universidad Autónoma de Madrid, 28049-Cantoblanco, Madrid, Spain

Detection of ionizing radiation is crucial in different fields including energy, national security, biological and nuclear research, etc. Some key characteristics of Single Wall Carbon Nanotubes (SWCNTs) is their metallic/semiconductor behavior, and conductive properties of the material, depends on the presence of defects in the structure. The presence of a defect in the nanotube walls, such as a single atom missing, incorporation or interaction with another structure, can result locally in the change of the electronic distribution and variation of the conductive characteristics of nanotubes. These defects, or interactions can be induced by the presence of ionizing radiation. The variations of these electrical properties can be measured, and a clear correlation can be established. Vertically Aligned Carbon Nanotubes (VA-CNTs) were grown by Chemical Vapor Deposition (CVD) on a silicon substrate with alternating layers of TiN and SiO2. VA-CNTs were exposed to X-ray radiation to study the change in resistivity. Preliminary results show an increase in the resistivity of CNTs as a function of radiation exposure time, meaning the structure is responding successfully to radiation exposure. The variation of resistivity has been associated with the presence of organic compounds that, during exposure to radiation, can generate species capable of interacting with the material by modifying its conductive properties. The first evidence indicates that the changes observed are reversible under heat treatment, which also supports the fact that it is the adsorbed organic species that, in the presence of radiation, modify the resistivity of the material and possibly allowing the material to be recyclable.

Biography

Kenneth Fontánez graduated from an AD in Mechanical Engineering at Universidad Ana G. Méndez-Gurabo, Puerto Rico, and is currently completing his studies in Chemistry. Kenneth Fontánez, a member of the Nanomaterials Research Group, is a research student who has worked on different projects related to hydrogen production by electrolysis, photocatalytic hydrogen production by water splitting, ionizing radiation sensors based on carbon nanotubes, development of anodes for lithium-ion batteries, and solar cells sensitized by dyes. Kenneth has presented his research work at multiple international conferences in the United States, Spain, Italy, etc.

Presenting author details

Kenneth Fontánez	(USA)

Oral Presentation

Contact Number: +1-939-645-0484

Email: kfontanez12@email.suagm.edu