Modeling and simulation of radiation effects on charge coupled devices induced by proton and neutron beams

Zujun WANG, PhD, State Key Laboratory of Intense Pulsed Irradiation Simulation and Effect, Northwest Institute of Nuclear Technology, CHN

Abstract

Charge coupled devices (CCDs) have extensive applications in particle detection, space mission, medical imaging, and nuclear industry. However, the CCDs used in these particular applications are susceptible to radiation damage. The radiation-induced damages can be separated into two phenomena: ionization damage effects and displacement damage effects. Ionizing damage induces an increase in trapped oxide charges and bulk-oxide interface traps which cause an increase in surface dark signals and dark signal non-uniformity (DSNU) of the CCDs. Displacement damage caused by energetic particles such as protons or neutrons induces stable bulk traps with energy levels within the band-gap which can lead to the degradations on the CCDs.

Fewer studies have focused on the details of simulation results and with generally only limited information about modeling and simulation process. A method is presented for modeling and simulation of radiation effects on CCDs irradiated by proton and neutron beams. The device model, clock pulse driver circuit models, radiation effect models, and solution methods are described in detail. The mechanism of dark signal and charge transfer inefficiency (CTI) increase induced by proton radiation is analyzed. The proton radiation modeling is established to simulate the degradation of dark signal and CTI in a CCD by the device simulator. The simulation results show that the CTI increase induced by protons is mainly due to the trap with energy level of Ec-0.42eV (E centers) and the total dark signal increase is dominated by the surface dark signal increase. The mechanism of CTI increase induced by neutron radiation is analyzed. The neutron radiation modeling is established to simulate the CTI degradation in a CCD. The tendencies of the simulation results are in agreement with the experimental results of the correlative literatures.

Recent publications (minimum 5)

- Zujun WANG (2016) Image analysis of single event transient effects on charge coupled devices irradiated by protons. Nuclear Instruments and Methods in Physics Research A 834:118-112.

- Zujun WANG (2015) Degradation of CMOS APS image sensors induced by total ionizing dose radiation at different dose rates and biased conditions, IEEE Transactions on Nuclear Science 62(2): 527-533

- Zujun WANG (2015) Displacement damage induce degradation of COTS array CCDs irradiated by neutron beams from a nuclear reactor. Nuclear Instruments and Methods in Physics Research A 771:49-54.

- Zujun WANG (2014) Dose Rate and Bias Effects on COTS Array CCDs Induce Dark Signals Increase. IEEE Transactions on Nuclear Science 61(3):1376-1380

- Zujun WANG (2014) Degradation of saturation output of the COTS array charge-coupled devices induced by total dose radiation damage. Nuclear Instruments and Methods in Physics Research A 751:31-35.

Biography (150 word limit)

Zujun WANG has his expertise in radiation effects and hardening technology of electrical devices. He is with the State Key Laboratory of Intense Pulsed Radiation Simulation and Effect, Northwest Institute of Nuclear Technology. He is working on the radiation effects on the photoelectric image sensors such as CCD and CMOS image sensors.

Email: wangzujun@nint.ac.cn