Experimental Evaluation of the Marcellus Shale Properties

Kashy Aminian, PhD, West Virginia University, USA

Samuel Ameri, West Virginia University, USA

### Abstract

Unconventional shale reservoirs play an enormous role in hydrocarbon production in the United States. Among the shale gas producing plays, Marcellus shale has a growing contribution due to advances in horizontal drilling and hydraulic fracturing techniques. Even though the advances in these technologies have unlocked the gas contained in Marcellus shale, the quantification of the petrophysical properties remain challenging due to complex nature of the shale. Reliable values of the shale petrophysical properties including permeability and porosity are necessary for accurate estimation of the original gas-in-place, prediction of the production rates, and optimization of the hydraulic fracturing treatments. Unsteady state methods have been extensively used to estimate permeability of the shale samples because the shales typically have permeability values in Nano-Darcy range. However, the permeability values by determined by these techniques have been found often to have large margin of uncertainty which are attributed to the lack of consistent experimental protocols and the interpretations issues.

In this study, Marcellus shale petrophysical properties were successfully measured using a laboratory set-up, referred to as PPAL (Precision Petrophysical Analysis Laboratory) here, which has been demonstrated to provide accurate and repeatable measurements of the shale petrophysical properties. The PPAL measurements are performed under steady-state isothermal conditions flow conditions and the analysis of the results do not require complicated interpretations such as those for pulse-decay or GRI methods. The key advantage of the PPAL is the capability to measure the permeability and porosity of the shale core plug under a wide range of confining and pore pressures.

The core plugs used in this study were made available through the Marcellus Shale Energy and Environment Laboratory (MSEEL). MSEEL is a dedicated field laboratory in the Marcellus Shale which has been established with purpose of providing a unique opportunity to undertake field and laboratory research to advance and demonstrate new subsurface technologies and to enable surface environmental studies related to unconventional energy development. The filed site contains several horizontal Marcellus Shale wells and a vertical well (science well) drilled specifically for obtaining core and log data for scientific purposes.

### Image

Figure 1. Schematic of PPAL

Recent Publications (minimum 5)

- Elsaig, M., Aminian, K., Ameri, S. and M. Zamirian, 2016:  “Study Analyzes Marcellus Shale Petrophysics,” American Oil & Gas Reporter, November Issue, pp. 63-65.

- Zamirian, M., Aminian, K. and S. Ameri, 2016: “Measuring Marcellus Shale Petrophysical Properties,” Paper SPE-174968180366, Proc. SPE Western Regional Conf., Alaska.

- Zamirian, M., Aminian, K., Ameri, S., 2015. “Measurement of Key Shale Petrophysical Properties.” Paper SPE 174968 Proc. SPE ATCE, Houston, Texas.

- Zamirian, M., Aminian, K., and Ameri, S., 2015. “Experimental Investigation of Geomechanical Impacts on Organic-Rich Shales Matrix-Fracture Characterization,” Paper SPE-1749681733028, Proc. SPE Eastern Regional Conference, Morgantown, WV.

- Zamirian, M., Aminian, K., Ameri, S., 2014. “New Steady-State Technique for Measuring Shale Core Plug Permeability.” Paper SPE-171613, Proc. Unconventional Resources Conference, Calgary, Canada.

Biography

Kashy Aminian, Professor of Petroleum & Natural Gas Engineering at West Virginia University, has 40 years of distinguished service in both industry and academia. He holds MS and PhD degrees both from University of Michigan. He has extensive research and teaching experience in the areas of unconventional natural gas resource development and reservoir engineering.

Email: kashy.aminian@mail.wvu.edu

Notes/Comments:

Presenting author details

Full name: Kashy Aminian

Contact number: 1-304-692-2278

Mail id: kashy.aminian@mail.wvu.edu

Category: (Oral presentation/ Poster presentation)