Characteristics of Over-Mature Marine Black Shale Reservoir in Lower Paleozoic, Sichuan, China

Tingshan ZHANG，Xiaohui CHEN

State Key Laboratory of Oil and Gas Reservoir Geology and Exploration, Chengdu 610500, China

School of Geoscience and Technology, Southwest Petroleum University, Chengdu 610500, China

Abstract

In order to interpret the reservoir characteristics of the Lower Paleozoic  black shale, eight core samples collected from Wufeng-Longmaxi formation were investigated using carbon sulfur analyzer, microphotometric, X-ray diffractometry (XRD), environmental scanning electron microscopy (ESEM) observation, mercury porosimetry, and low pressure nitrogen adsorption methods.. Results indicate that the TOC content of shale samples range from 0.51% to 5.91%, and their equivalent vitrinite reflectance (VRr) ranges from 2.79% to 3.04%. The total porosity of the black shale samples ranges from 3.58% to 4.85%, and their average pore-throat diameter of macropores ranges from 0.05 to 0.1μm. Low pressure N2 adsorption isothermals indicate the total surface areas (SBET) ranges from 9.55 m2/g to 18.89 m2/g, and pore volume ranging from 0.011 cm3/g to 0.020 cm3/g. In addition, the total porosity, the total surface areas and the pore volume show good positive relationship with TOC and weak positive relationship with total clay mineral content. The average pore size of eight samples ranges from 2.92 nm to 3.68 nm. The TOC content,keroganic type ,clay mineral and thermal evolution are believed to be the main factors to control the pore structure development. This study demostrates some important factors of the over-mature Wufeng-Longmaxi black shale reservoir, and they could be helpful to the exploration activity and reservoir evaluation of the study area.

Recent Publications

- Loucks, R. G., Reed R. M, Ruppel S. C., Jarvie D. M., 2009. Morphology, genesis, and distribution of nanometer-scale pores in siliceous mudstones of the Mississippian Barnett shale. Journal of Sedimentary Research, v. 79, p. 848-861. DOI: 10.2110/jsr.2009.092

- Chalmers G. R., Bustin R. M., Power I. M., 2012. Characterization of gas shale pore systems by porosimetry, pycnometry, surface area, and field emission scanning electron microscopy/transmission electron microscopy image analyses: Examples from the Barnett, Woodford, Haynesville, Marcellus, and Doig units. AAPG Bulletin, v. 96, no. 6, pp. 1099-1119. DOI: 10.1306/10171111052

- Kuila U., and Prasad M., 2013. Specific surface area and pore-size distribution in clays and shales. Geophysical Prospecting, v. 61, pp. 341-362. Doi: 10.1111/1365-2478.12028

- Guo T., Zhang H., 2014. Formation and enrichment mode of Jiaoshiba shale gas field, Sichuan Basin. Petroleum Exploration and Development, v. 41, no. 1, pp. 28-36. (in Chinese with English abstract)

- Tian H., Pan L., Xiao X., Wilkins R. W.T., Meng Z., Huang B., 2013. A preliminary study on the pore characterization of Lower Silurian black shales in the Chuandong Thrust Fold Belt, southwestern China using low pressure N2 adsorption and FE-SEM methods. Marine and Petroleum Geology, v. 48, pp. 8-19. http://dx.doi.org/10.1016/j.marpetgeo.2013.07.008

Tingshan Zhang works as a professor and Ph.D. supervisor at Southwest Petroleum University, Chengdu, Sichuan province,China. His research interests include paleobiology, microbial geology, and unconventional oil and gas reservoirs. He earned his bachelor’s degree and doctorate of science from the Department of Geology,Nanjing University, Nanjing, China, in 1982 and 1997, respectively.

Email: zts_3@126.com