Investigation on high-energy ultra-short mid-infrared coherent radiation generation from OPA in BGSe crystal

Feng Yang, PhD, TIPC, China

Abstract (250 word limit)

Powerful and compact coherent radiation source with wavelength covering the 3-14µm mid-Infrared spectral range is crucial for a number of applications. The parametric frequency down-conversion pumped by the mature and efficient 1µm  laser in IR nonlinear optical crystals have been proved to be an effective way to generate this tunable coherent radiation. Here we present a high energy widely and continuously tunable picosecond coherent radiation source from an optical parametric amplifier (OPA) pumped by1064nm Nd:YAG laser in the newly developed BaGa4Se7 (BGSe) crystal. In experiment, the 3-5µm tuning with output energies in the 1mJ-300µJ range , 6.4-11µm tuning with output energies in the 100-38µJ range and 8-14µm tuning with output energies in the 230-140µJ range were demonstrated, respectively. Further, the type-I (eeo) phase-matching angles of BGSe crystal for mid-Infrared light generation were experimentally measured. The nonlinear absorption of BGSe crystal under 1064 nm pumping was investigated for the first time. The results confirm the great potential of BGSe in ultra-short mid-Infrared tunable coherent radiation generation owing to its wide transmission range, high laser damage threshold, 1µm laser pumping and large size. On the other hand, it exhibits a strong nonlinear loss under 1µm laser pumping intensity >1.3GW/cm2, which limits the generation of high energy ultra-short mid-Infrared laser to some extent.

Image

Figure 1: Experiment scheme of the optical parametric amplifier based on BGSe crystal.

Recent Publications (minimum 5)

- J. Y. Yao et al. (2010) BaGa4Se7: A New Congruent-Melting IR Nonlinear Optical Material. Inorg. Chem. 49 (18):9212-9216.

- F. Yang et al. (2013) High efficiency and high peak power picosecond mid-infrared optical parametric amplifier based on BaGa4Se7 crystal.  Opt. Lett. 38 (19):3903-3905.

- F. Yang et al. (2015) Mid-infrared Optical Parametric Amplifier With 6.4-11 μm Range Based on BaGa4Se7. IEEE. Photo. Tech. Lett. 27 (10): 1100-1103.

- K. Kato et al. (2017) Phase-matching propertise of BaGa4Se7 for SHG and SFG in the 0.901-10.5910 μm range. Appl .Opt. 56 (11): 2978-2981 .

- N. Y. Kostyukova et al. (2017) Widely tunable in the mid-IR BaGa4Se7 optical parametric oscillator pumped at 1064 nm. Opt. Lett. 41 (15): 3667-3670.

Biography (100 word limit)

Feng Yang has his expertise in all-solid-state lasers and characterization of nonlinear optical materials. He received the Ph.D. degree in optical physics from the Institute of Physics, Chinese Academy of Sciences (CAS) in 2010. He is currently an Associate Professor in the Technical Institute of Physics and Chemistry (TIPC) in CAS. He has coauthored approximately 150 papers in scientiﬁc journals and held about 20 patents. The researches on ultraviolet and vacuum ultraviolet laser, mid-infrared laser and new nonlinear optical materials have been carried out.

Email: yang12345yang@163.com

Notes/Comments: