# Extending petroleum resources – substitution of petro-products from Glycerol

A.K. Suresh, PhD, Professor of Chemical Engineering, IIT Bombay, Mumbai, India 400076

### Abstract

### Glycerol, because of its multi-functionality, is a potential source of many chemicals1 currently produced from petroleum sources. Being a byproduct of biodiesel manufacture, the availability of glycerol has been on the increase, but the same fact results in price fluctuations, which means that the best strategies to valorize glycerol must concentrate on high value products. Here, we discuss two possibilities for the upgradation of glycerol – via oxidation to glyceric and tartronic acids, and via hydrogenolysis to 1,3-propanediol. Glycerol oxidation, while a potential route to several high value chemicals, is a complex reaction2, and directing it to desired compounds is a challenge. Catalysts such as Pd, Pt and Au, under alkaline conditions, favour the formation of glyceric and tartronic acids, but carbon chain scission leading to C2 and C1 compounds limits the selectivity. Both rate and selectivity of such catalysts are modulated in interesting ways by the catalyst support3, with carbon supports providing the best performance. An analysis of the product spectrum under different reaction conditions suggests ways of reaction network reduction4, based on which the network has been modeled, and the parameters evaluated for different catalysts. The consequences of possible transport limitations at the catalyst and/or the macro-level in a three phase reactor, especially on the selectivity of the reaction, have been evaluated.

### Hydrogenolysis of glycerol to 1,3-propanediol, precursor to poly (trimethylene) terephthalate, is again, a nonselective reaction since it involves attacking the secondary hydroxyl group of glycerol while leaving the two primary hydroxyls untouched. Multi-step chemical routes have been proposed5, but yields have been poor.

We have been experimenting on a strategy of chemically blocking the primary hydroxyls through derivatization, eliminating the secondary alcohol group, and de-derivatizing. The second step can be performed catalytically or non-catalytically. Some of the challenges associated with the reaction will be discussed on the basis of the results from our work so far.

Recent Publications

- Werpy T, Petersen G. (Eds) (2004) Top value added chemicals from Biomass vol I. USDoE report.

- Katryniok B, Kimura H, Skrzynska E, Girandon J-S, Fongarland P, Capron M, Ducoulombier R, Mumura N, Paul S, Dumeignil F (2011) Selective catalytic oxidation of glycerol: perspectives for high value chemicals. Green Chemistry 13:1960-1979.

- Ashutosh Namdeo, Mahajani SM and Suresh AK (2016) Palladium catalyzed oxidation of glycerol – effect of catalyst support. J. Mol. Cat. A: Chemical 421:55-56.

- Ashotosh Namdeo (2016). Catalytic studies on liquid phase glycerol oxidation. PhD thesis, IIT Bombay.

- Wang K, Hawley MC, DeAthos SJ (2003) Conversion of glycerol to 1,3-propanediol via selective dehydroxylation. I & EC Research

42: 2913-2923.

Biography

Prof A.K. Suresh completed his PhD from Monash University. After a brief stint with Hindustan Lever Research Centre (a Unilever company), he joined the Indian Institute of Technology Bombay in 1988 and has been with the Institute ever since. He currently holds an Institute Chair in the Department of Chemical Engineering. He served as the Head of the department from 2005-2008, and as the Dean of Faculty Affairs of IIT Bombay during 2009-2014. Apart from an abiding interest in liquid phase hydrocarbon oxidations, Prof Suresh’s work encompasses other themes in transport and reaction engineering. He is a Fellow of the Indian National Academy of Engineering.

Email: aksuresh@iitb.ac.in