STUDY OF CHEMICALLY SYNTHESIZED RARE-EARTH TERBIUM SUBSTITUTED BAM HEX FERRITES

Submitted by

ENGR. MUHAMMAD IMRAN ASGHAR

DR. MOHAMMAD YOUNAS

(Associate Professor)

### Abstract

The current study focuses on Barium based rare-earth terbium (Tb3+) substituted M-type hexagonal ferrite materials with nominal composition BaTbxFe12-xO19 (x=0.00, 0.10, 0.20, 0.30) synthesized using Sol-gel Auto-combustion technique. The rare-earth terbium (Tb3+) element has been substituted at Fe3+ site to investigate its effect on chemical, electrical and magnetic characteristics of M-type hexagonal ferrite. All the prepared samples were sintered at temperature 1000 ◦C as estimated from the DSC-TGA analyses to obtain the single phase ferrite component.

The labelled peaks appeared in different EDX spectra confirm the presence of Ba, Tb, and Fe elements in the synthesized compounds. The concentration of (Tb3+) increases, while that of Fe3+ decreases, which shows the replacement of rare-earth terbium (Tb3+) ions with that of  Fe3+ ions. XRD analysis confirms the successful formation of single phase rare-earth terbium (Tb3+) substituted ferrite materials. This crystallite range was set up to fall near the value of 65.15 - 37.35 nm, that is suitable to achieve the best signal of noise for recording components. Scanning Electron Microscopy laboratory analysis exhibits the geometry of grains in hexagonal shape. X-ray density improves from 8.084 - 8.359 g/cm3 and the bulk density enhances from 5.78-5.93 g/cm3.

Keywords:  Rare-earth; Sol-Gel Auto-combustion; BaFe12O19; Chemical co-precipitation; Chemical homogeneity.

Biography

Muhammad Imran Asghar worked as process engineer in sugar and glucose production units. Now

working as Senior Instructor , Chemical Technology Department in Government College of Technology

Kamalia. My work in this thesis is to chemically synthesized the rare earth terbium substituted BaM

Hexa ferrites

.

Email Address:   imran1097@yahoo.com

+923336558531

# REFRENCES

[1]	P. J. Kelly and R. D. Arnell, Vol. 56, 159 (2000).

[2]	B. Viswanathan, V.R.K. Murthy, “Ferrite Materials; Science and Technology”, Narosa publishing house, New Delhi, 1990, p. 429.

[3]	E.C. Snelling, “Soft Ferrites/Properties and Application”, second ed, Butter worths, London, 1988, 1.

[4]	T.L. Hylton, M.A. Parker, M. Ullah, K.R. Coffey, R. Umphress,J.K. Haward, J. Appl. Phys. 75 (1994) 5960.

[5]	 A. Morisako, M. Matsumoto, M. Naoe, IEEE Trans. Magn. 24 (1988) 3024.Holland, Amsterdam, 1982, p. 305.